Contents lists available at ScienceDirect

Computers and Electrical Engineering journal homepage: www.elsevier.com/locate/compeleceng

| # A novel approach to low-light image and video enhancement using adaptive dual super-resolution generative adversarial networks and top-hat filtering Vishalakshi, Shobha Rani, Hanumantharaju ∗ Research center: Department of Electronics and Communication Engineering, BMS Institute of Technology and Management, Bengaluru, India Visvesvaraya Technological University, Belagavi, Karnataka, India |  |  |  |
| --- | --- | --- | --- |
| A R T I C L E I N F O |  | A B S T R A C T |  |
| Keywords: Adaptive fusion Generative adversarial network Image and video enhancement Mobile devices Super-resolution Top-hat transform |  | Image and video enhancement under low-light conditions is challenging, as the task involves more than just brightness adjustment. Without addressing issues such as artifacts, distortions, and noise in dark regions, brightness improvement alone can worsen the quality. This paper presents a novel approach to low-light image and video enhancement based on the adaptive fusion of Dual Super-Resolution Generative Adversarial Network (DSRGAN) models, followed by Top-Hat Gradient-Domain Filtering (THGDF). A soft thresholding mechanism is used to integrate the Memory Residual Super-Resolution Generative Adversarial Network (MRSRGAN) and the Weighted Perception Super-Resolution Generative Adversarial Network (WPSRGAN). MRSRGAN enhances fine details, improving the objective performance of the image, while WPSRGAN improves overall details, enhancing the subjective performance. Top-hat gradient- domain filtering is then applied to remove artifacts, distortions, and noise in both images and videos, resulting in outstanding perception scores. The proposed approach is validated using the quality assessment metrics such as Peak Signal-to-Noise Ratio (PSNR), Structural Similarity Index Measurement (SSIM), and Information Fidelity Criterion (IFC). Extensive experiments conducted on publicly available source codes and databases demonstrate that the proposed method is more effective than the existing state-of-the-art techniques. |  |

## 1.  Introduction

Capturing clear and detailed images or videos in low-light conditions is challenging. Due to inadequate lighting, noise levels increase, degrading the overall image quality. Additionally, long exposure times used to gather more light can introduce motion blur, affecting the sharpness of the captured image. The reduced dynamic range also results in overexposed highlights and underexposed shadows. These issues have been effectively addressed by numerous researchers using deep learning models [1–3], video stabilization techniques  [4], video restoration methods  [5], and more. Although, these approaches yield improved results, video enhancement under extreme low-light conditions with artifacts, distortions and noise remains a challenge.

Existing research has introduced promising model-based schemes [6] and data-driven methods [1] to enhance images and videos. Conventional mathematical approaches can improve resolution, contrast, brightness, and color under moderate lighting conditions. However, enhancing the quality of an image and video under low-light conditions, especially in the presence of noise, remains

∗ Corresponding    author    at:    Research    center:    Department    of   Electronics    and    Communication    Engineering,    BMS   Institute    of    Technology    and Management, Bengaluru, India.

E-mail address:   mchanumantharaju@bmsit.in  (Hanumantharaju).

https://doi.org/10.1016/j.compeleceng.2024.110052

Received 31 July 2024; Received in revised form 15 December 2024; Accepted 29 December 2024 Available online 13 January 2025

0045-7906/© 2025 Elsevier Ltd. All rights are reserved, including those for text and data mining, AI training, and similar technologies.

an  area  for  improvement.  Image  and video  enhancement,  particularly  reconstruction  under noisy  conditions,  continues  to be  a challenge.  These  enhancement  techniques  improve  the  viewing  experience  both  visually  and  informatively  through  operations such  as  temporal  filtering,  super-resolution,  color  correction,  contrast  adjustment,  deblurring,  and  noise  reduction.  Image  and video enhancement techniques are widely used across various scientific and commercial applications. Low-light enhancement has a wide range of applications, including video surveillance  [7], autonomous driving  [8], medical imaging  [9], and computational photography  [10].

Smartphone users face significant challenges in improving low-light images and videos due to limited hardware capabilities, processing  power,  and  high  user  expectations.  Limited  aperture  sizes  on  mobile  devices  result  in  insufficient  light  capture, making low-light enhancement even more difficult. Real-time processing requirements, along with the limited computing power of smartphones, necessitate efficient algorithms that balance performance with speed. Memory constraints also limit the complexity and size of models that can be employed for these enhancements. Hence, algorithm optimization is crucial to meet user expectations. The need for seamless image and video enhancement under low-light conditions is especially immediate requirement for the smartphone users. Balancing technical challenges with the demand for high-quality performance is critical for improving mobile phone stored images and videos in low-light conditions.

The primary goal of low-light image enhancement (LLIE) is to improve the interpretability and perception of images captured in low-light conditions. In recent years, deep learning-based solutions incorporating various learning strategies, network architectures, loss functions, and training data have gained significant attention in this domain. Advances in deep learning and computer vision have led to the continuous development of sophisticated video enhancement approaches. It is likely that these methods will soon be implemented via user-friendly web interfaces and unified online platforms, which will include various popular LLIE techniques. Research in this area is increasingly important as efforts continue to address the challenges of low-light enhancement for smartphone applications.

Conventional low-light image and video enhancement methods include histogram equalization (HE) and retinex-based models. Among  these,  retinex-based  methods  have  gained  the  most  attention  in  recent  research.  Many  researchers  have  contributed  to developing  retinex-based  deep  learning  models  [11,12].  Retinex-based  approaches  typically  decompose  low-light  images  into reflection and illumination components using priors and regularization techniques, with the enhanced result being the estimated reflection  component.  However,  retinex-based  methods  have  limitations.  In  complex  lighting  conditions  or  extremely  low-light situations, color balance and detail enhancement are often compromised. Additionally, these methods can introduce artifacts or unrealistic enhancements that need to be addressed in future LLIE developments to create more robust solutions.

One of the most popular techniques for restoring images under low-light conditions is single image super-resolution (SISR) [13]. Super-resolution (SR) algorithms reconstruct HR images from corresponding low-resolution (LR) images. Deep learning algorithms typically perform faster and yield better results than traditional SR algorithms, as demonstrated by various performance metrics such as PSNR, SSIM, and IFC. Deep learning-based SR algorithms excel at visual enhancement. According to the literature, SR algorithms can be divided into two categories. In the first category, Convolutional Neural Networks (CNNs) are used as optimization functions with traditional L1 or L2 loss functions in pixel space. In the second category, Generative Adversarial Networks (GANs) are employed, introducing a novel approach to image reconstruction.

Lintao  et  al.  [14]  introduced  a  novel  framework  for  low-light  image  enhancement  that  synthesizes  normal  and  low-light image pairs to enhance training data. By employing an illumination degradation model and a self-attention-based enhancement model,  the  approach  improves  robustness  and  performance  across  various  tasks,  outperforming  existing  methods.  It  effectively augments data, demonstrates superior performance, and improves robustness in complex scenes. However, it lacks discussion on computational complexity and the results are not satisfactory with generalization in highly varied environments. Kumar et al.  [15] presents the Morphological Transformation-based Locality-Constrained Representation (MTLcR) model for low-light robust face SR. This  approach  detects  low-light  effects  in  input  images  using  domain-specific  dictionary  images,  applying  similar  effects  to  LR dictionary images to minimize reconstruction errors. The incorporation of morphological transformations into the objective function enhances reconstruction weights, resulting in improved performance. Simulation results on publicly available datasets and real-world images demonstrate the model’s superior reconstruction capability under low-light conditions compared to state-of-the-art methods. However, the paper lacks with a detailed analysis of computational complexity and scalability in diverse scenarios.

### 1.1.  Contributions

This paper presents DSRGAN, a novel SR framework for low light image and video enhancement. The main contributions of the proposed method work as mentioned below:

a.  Objective The primary objective of this paper is to address the challenges of enhancing image and video quality under low- light conditions. The authors recognize that merely adjusting brightness is insufficient; the enhancement process must also tackle artifacts, distortions, and noise prevalent in dark regions. This work proposes a novel method that utilizes advanced GANs  and Top-Hat  Gradient-Domain  Filtering  techniques  to  improve  the perceptual  and  objective  quality  of images  and videos captured in low-light environments.

b.  Data Collection and Preprocessing In this work, the sources of images and videos are collected from five publicly available benchmarks  for  image  SR:  Sun-Hays,  Urban100,  Set5,  Set14,  and  a  mobile  dataset.  The  Sun-Hays,  Urban100,  Set5,  and Set14 datasets primarily consist of natural images, while the mobile dataset contains images captured from various device models, including the iPhone 3GS, BlackBerry Passport, Sony Xperia Z, and Canon 70D DSLR, all of which are freely available

Fig.  1.  An  illustration  of railway  station  image  reconstructed  using  existing  methods  and  its  comparison  with  the  proposed  method.(a)  Low  Resolution  Input Image (PSNR:22.87, SSIM:0.6, IFC:1.8) (b) Ground Truth Image (c) BGAN  [16] (23.47, 0.67, 1.89), (d) RGSR  [17] (23.45, 0.65, 1.76), (e) BDRVSR  [18] (23.51, 0.61,  1.81),  (f)  RESRGAN  [19](23.56,  0.64,  1.79),  (g)  DBVSR  [20]  (23.66,  0.68,  1.82),  (h)  BlindSR  [21]  (23.39,  0.60,  1.85),  and  (i)  Proposed  Method  (23.7, 0.69,  1.98).

for public download. Preprocessing steps include normalization of pixel values, data augmentation through techniques like rotation and cropping, and the generation of LR versions of HR images, ensuring the input data is suitable for effective training and evaluation of the proposed methodologies.

c.  Methodology The methodology consists of an adaptive fusion approach combining DSRGAN with THGDF. The DSRGAN models are designed to enhance fine and overall details in the images. A soft thresholding mechanism is employed to integrate the outputs ofthe MRSRGAN, which focuses on detail enhancement, and the WPSRGAN, which aims to improve overall visual quality. The THGDF is then applied to address and eliminate any residual artifacts, distortions, and noise, ensuring a visually appealing output.

d.  Feature Learning The feature learning of the model includes specific settings to effectively train and evaluate models. The training dataset comprised Flickr2K, with 2,650 HR images, while evaluation datasets included Urban100, Sun-Hays, Set5, and Set14, providing over  100 images for assessment. The LR images are generated using bicubic downsampling, with a LR patch size of 24 × 24 pixels and an HR patch size of 96 × 96 pixels. Data augmentation techniques, including random horizontal and vertical flipping, enhanced model robustness, with a batch size set to 16. For the MRSRGAN, the generator architecture featured 7 residual blocks with a filter size of 3 × 3 pixels. The learning rate was initialized at  1 × 10_4 using the Adam optimizer, trained over 500 epochs. In the WPSRGAN, the generator had 16 residual blocks and utilized a PReLU activation function.

e.  Evaluation Metrics The performance of the proposed method is assessed using standard quality assessment metrics, including PSNR, SSIM, and IFC. These metrics provide quantitative measures of image quality and enable comparison with existing methods. The paper presents detailed evaluations demonstrating how the proposed technique achieves higher scores in these metrics, further validating its effectiveness in low-light image and video enhancement.

f.  Results Numerous experiments were conducted to evaluate the performance of the proposed approach against existing state- of-the-art techniques. The benchmark methods used are BGAN [16], RGSR [17], BDRVSR [18], RESRGAN [19], DBVSR [20], and BlindSR  [21]. The experimental results presented demonstrate that the proposed method is more effective compared to other state-of-the-art methods.

Fig. 1(a) shows a low-light  image generated from the ground truth or high-resolution  (HR) image.   Fig. 1(b) shows the HR image.  Figs. 1 (c) through (h) depict results from other SR methods such as BGAN [16], RGSR [17], BDRVSR [18], RESRGAN [19], DBVSR [20], and BlindSR [21]. The result of the proposed method is shown in  Fig. 1(i). As it is evident from the PSNR, SSIM, and IFC metric comparisons, the proposed method reconstructs images more effectively than other existing techniques.

The rest of the paper is organized as follows:  Section 2 reviews the existing work in four stages, outlining the contributions made by other researchers.  Section 3 details the proposed approach.  Section 4 presents the experimental results of the proposed method on  standard  datasets. The results  demonstrate  that  the proposed method produces more visually  appealing, vivid,  and brilliant images and videos compared to other competitive methods. Although the results presented focus on images, the proposed method has also been tested on standard video datasets. Finally,   Section 5 summarizes the key contributions of this work.

## 2.  Background and related works

Several SISR algorithms based on deep learning have been proposed by various researchers, showing significant performance improvements  [22,23]. This paper focuses on five key aspects in the literature review: SR using Convolutional Neural Networks (SRCNN)  [24], Very  Deep  SR  (VDSR)  [25],  Enhanced  Deep  SR  (EDSR)  [26],  SR using  GANs  (SRGAN)  [27],  and  image  quality evaluation.

### 2.1. Super-resolution convolutional neural network (SRCNN)

SRCNN is a standard in image SR that demonstrates how deep learning can be applied to enhance image quality. Dong et al. [24] introduced SRCNN, which is designed to directly learn the complex mapping between LR and HR images. Feature extraction is

accomplished through three convolutional layers. Unlike other artificial  intelligence  (AI) techniques,  SRCNN operates on image patches, allowing it to focus on local attributes within the data. As a supervised learning algorithm, SRCNN optimizes its parameters using  paired  LR  and  HR  images  to  minimize  the  difference between  predicted  and  ground  truth  images.  SRCNN  offers  several compelling  advantages  compared  to  other  existing  methods.  Firstly,  its  architecture  is  designed  for  simplicity  while  achieving superior accuracy compared to state-of-the-art example-based methods. Secondly, it employs a moderate number of filters and layers, enabling SRCNN to operate efficiently in practical online applications, even on a CPU. SRCNN outperforms a series of example- based methods due to its fully feed-forward nature, eliminating the need to solve optimization problems during runtime. Thirdly, experimental work of other investigation shows that enhancing the network’s restoration quality is feasible with larger datasets and/or a larger model. In contrast, larger datasets or models can pose challenges for existing example-based methods.

The initial step in SRCNN involves up-scaling a single LR image to the desired size using bi-cubic interpolation. This serves as the pre-processing step. The resulting interpolated image is denoted as Y. The objective of SRCNN is to recover from Y an image F(Y) that closely resembles the ground truth HR image. For clarity, Y is referred to as an LR image, even though its size matches that of the HR image X. The model proposed by Lu et al.  [28], based on shuffle learning, is designed for deployment on mobile devices with limited computing power. Through extensive experiments on three widely used datasets, Lu’s model not only achieves high PSNR and SSIM but is also easier to implement compared to other image super-resolution (SR) models due to its simplicity.

Many  applications  require  HR  images  because  they  include  complementary  information.  However,  improving  pixel  density through sensor and visual technologies is limited and costly. Elgohary et al.  [29] used image processing methods to generate HR images from their LR counterparts to overcome this challenge. Video Super-Resolution (VSR) aims to recover complex details and mitigate sensor effects. The analysis utilizes wavelet analysis and CNNs to combine multi-frame SR with wavelet analysis, resulting in a multiscale representation of video frames obtained through wavelet decomposition. Consequently, several CNNs are trained independently to approximate wavelet multiscale characteristics. Inference on these trained CNNs is conducted through regression of wavelet multiscale representations from LR frames. Additionally, wavelet reconstruction is performed, resulting in reconstructed HR frames. The research comprises the use of various camera zoom lenses to capture the LR frames and create HR frames. The authors conducted several experiments demonstrating the effectiveness of their approach in restoring video frames.

The review of existing work shows that CNNs have demonstrated exceptional performance in SR techniques. However, these models  may  not  be  feasible  for  low-cost  devices.  The  Multiscale  Feature  Selection  Network  (MFSN),  introduced  by  Minghon et al.  [30], addresses this challenge. The Multiscale Feature Selection Block (MFSB), at the core of MFSN, is developed to extract rich multiscale features from coarse-to-fine receptive fields. In each section of MFSB, excluding the last, a wide-activated residual unit is employed to enhance representation  capability. By adaptively varying the receptive field sizes of two adjacent divisions within MFSB, the Scale Selection Module  (SSM) efficiently merges features from these branches. Furthermore, a Comprehensive Channel Attention Mechanism (CCAM) is seamlessly integrated into SSM, dynamically adapting selection weights based on local and global inter-channel dependencies. The results presented by the authors show that MFSN compares favorably with other lightweight methods. SR is inherently a multi-solution process due to its one-to-many mapping from LR to HR space. However, existing SR networks treat the SR of different sizes for the same image as independent tasks, making it challenging to leverage feature learning experiences and requiring extensive computational time to recover HR images. These issues also exist in current arbitrary-scale SR methods.

Wenyuan et al.  [31] introduced a novel self-generating (SG) multiscale learning method based on SG mechanisms to achieve efficient and effective HR image recovery. The SG-SR method uses feature learning outcomes from SR networks to generate upscale filters as an alternative to traditional upscale modules. As each scale factor is adjusted, the SG upscale module provides weights to filter the LR tensor, resulting in HR images corresponding to the filtered tensors. Based on numerous experiments and comparisons of benchmark datasets, Wenyuan’s method outperforms state-of-the-art methods. Additionally, the SG upscale module significantly reduces computational costs in association with other upscale modules while enhancing the effectiveness of existing SR networks.

By integrating explicit feature learning into CNNs, Qizhou et al.  [32] enhance the representation capabilities of CNNs through Trainable Feature Matching (TFM). The TFM features can be explicitly learned from training images through feature matching by incorporating trainable feature sets. Qizhou’s Trainable Feature Matching Attention Network (TFMAN) improves SR performance by  incorporating  non-local  and  channel  attention.  Due  to  computational  demands,  the  authors  introduce  a  streamlined  variant called Same-size-divided Region-level Non-Local (SRNL), which performs non-local computations in parallel on uniformly divided blocks from the input feature map. The effectiveness of SRNL and TFM is validated through ablation studies, with comprehensive experimental results from benchmark datasets showing that TFMAN consistently outperforms other methods.

The review of existing methods shows that  SRCNNs are prevalent in image  SR.  CNN structures  can be adaptively modified, and advanced self-attention mechanisms can be incorporated to enhance SR performance. A common characteristic among these improvements is that CNNs implicitly learn feature processing modes resembling convolution and non-local operations instead of explicitly focusing on high-frequency details. Conversely, a dictionary-based approach explicitly learns feature decompositions to match and reconstruct LR features (see Table 1).

### 2.2.  Very deep super-resolution (VDSR)

Through deep CNNs, Very Deep Super-Resolution (VDSR) advances the boundaries of resolution enhancement in image SR. VDSR, initially developed by Kim et al.  [25] in 2016, which incorporates a deeper network design to capture more complex and subtle features inherent in HR images. VDSR can learn complex mappings between LR and HR image spaces   by leveraging significantly

Table 1

Summary of SRCNN Methods.

| Method | Key Features | Strengths | Limitations |
| --- | --- | --- | --- |
| SRCNN [24] | Uses deep learning to | Simple architecture with | Requires paired LR-HR |
|  | directly map LR to HR | high accuracy. Efficient for | images. Performance can |
|  | images through 3 | online applications, even on | improve with larger datasets |
|  | convolutional layers. | CPU. | and models, which may not |
|  | Performs image patch extraction. |  | always be feasible. |
| SISR [28] | Model designed for mobile devices with limited computing power. Simple architecture with high PSNR and SSIM. | Easy implementation with high performance on standard datasets. | May not scale well to larger or more complex datasets. |
| WCNN VSR [29] | Combines wavelet analysis | Effective for video SR and | Computationally intensive |
|  | with CNNs for multi-frame | mitigating sensor effects. | for video data, and requires |
|  | SR. Performs wavelet |  | specialized camera |
|  | decomposition and |  | equipment for capturing LR |
|  | reconstruction for HR video frames. |  | frames. |
| MFSN [30] | Features a Multiscale Feature Selection Block (MFSB) and a Comprehensive Channel Attention Mechanism (CCAM) for extracting rich features. | Lightweight and effective for low-cost devices. Dynamically adjusts receptive field sizes and merges features efficiently. | May not be as effective for very high-resolution images or large-scale datasets. |
| SG-SR [31] | Uses self-generating | Significantly reduces | May be less flexible in |
|  | multiscale learning to create | computational cost while | handling highly varied |
|  | upscale filters, reducing | maintaining SR effectiveness. | image types or real-time |
|  | computational costs while | Outperforms existing | applications. |
|  | enhancing SR network | methods in terms of |  |
|  | performance. | efficiency and performance. |  |
| TFMAN & SRNL [32] | Incorporates Trainable | Enhances feature matching | Computationally demanding |
|  | Feature Matching (TFM) and | with non-local attention, | for larger datasets. The |
|  | non-local channel attention | improving SR performance. | effectiveness of SRNL is still |
|  | to improve SR performance. | Streamlined for | reliant on the quality of the |
|  | Introduces a streamlined | computational efficiency. | feature extraction process. |
|  | version (SRNL) for parallel computation. |  |  |

increased convolutional layers. With residual learning, VDSR streamlines the optimization process. VDSR enhances performance by learning residuals, the difference between LR inputs and their corresponding HR targets.

VDSR achieves notable improvements in SR tasks, enabling the reconstruction of fine details and textures in HR images due to its deep architecture and residual learning principles. In addition to learning complex feature representations through multiple layers of abstraction, VDSR effectively captures complex image characteristics. VDSR can understand complex feature representations through various layers of abstraction. Further, VDSR’s usage and popularity highlights the importance of architectural innovations in deep learning. VDSR demonstrates how deeper networks can unlock new performance levels for various computer vision tasks. Through deep learning methodologies, VDSR represents a fundamental step forward in pursuing super-resolution images. It was observed in  Fig. 2 that in all cases the network trained with depth-wise fine-tuning converges more quickly, which support our hypothesis. Notably, both the speed of convergence and the restoration performance have been substantially improved. This improvement is especially evident with deeper neural networks, indicating the potential for depth-wise fine-tuning to enhance training stability.

Diffusion CNNs (DCNNs) are widely used to extract features and reconstruct details for SISR. However, many existing DCNN methods  cannot  fully  leverage  the  complementary  strengths  of feature  maps,  channels,  and  pixels.  This  limits  their  ability  to represent abundant features. Therefore, the specified problem is efficiently addressed using the Weidong et al. [33] Cascaded Visual Attention Network (CVANet) method. CVANet focuses on reconstructing details and is inspired by the visual attention mechanism of the human eyes. CVANet comprises a trainable Feature Attention Module (FAM) for learning feature-level attention, a channel attention module for improving feature maps by learning channel-level attention, and a pixel attention module for generating HR images. This is accomplished by adaptively selecting representative features from preceding layers. CVANet consistently outperforms state-of-the-art methods regarding subjective visual perception, PSNR, and SSIM on four benchmarks. The authors have provided CVANet’s code using the following URL: https://github.com/WilyZhao8/CVANet.

Medical  imaging,  especially  fluorescence  and  ultrasound  captures,  requires  advanced  solutions  to  track  molecular  dynamics accurately in real-time in cellular environments. In this context, deep learning-based image denoising offers significant improvements over conventional methods. The Multiscale Self-Attention Network (MSAN) is a new architecture proposed by Kyungsu et al.  [34] for optimal denoising of fluorescence and ultrasound images. An MSAN consists of a feature extraction layer to identify high and

Fig. 2.  VDSR results at various depths. Dotted lines represent the baseline models, while solid lines indicates the depth-wise fine-tuned networks.

low-frequency attributes, a multiscale self-attention mechanism for predicting residuals from original and downscaled feature maps, and a residual image decoder. A denoised image is obtained by offsetting the residual output. Compared to state-of-the-art models such as RIDNet  [35] and DnCNNp  [36], MSAN achieves notable PSNR improvements for the Fluorescence Microscopy Denoising (FMD)  [37], W2S  [38], and ultrasound datasets. It is clear from the specified method that MSAN effectively improves denoising capabilities for fluorescence and ultrasound images in real-time, enabling the decoding of complex biological processes.

Yeh et al. [39] aim to ease temporal flickering when upscaling videos by Fine-Grained VSR (FGVSR). However, several existing methods need help to utilize spatial–temporal information effectively from LR videos, resulting in unwanted artifacts or inaccurate reconstructions of the image’s details. To overcome these issues, the author proposed a deep learning framework for FGVSR, which takes consecutive LR video frames as input and generates SR frames corresponding to those frames. The framework emphasizes the alignment and refinement of multiple frames to reconstruct missing information from LR sources. An alignment module is introduced to align numerous frames at the feature level, preventing flickering in the video output. A feature fusion module is used once the aligned features have been fused and refined. On top of that, a refinement module is presented to reconstruct missing information based on the fused features. An image enhancement module is embedded in the skip connection for further refinement. The authors showed that deep FGVSR outperforms existing deep learning-based VSR methods. Detailed experiments are performed on REDS, Vid4, and Vimeo90k benchmarks, achieving state-of-the-art performance. In comparing FGVSR and state-of-the-art VSR methods, FGVSR shows that PSNR has substantially improved.

With  the  rise  of deformable  alignment-based  VSR  methods, VSR has  gained  significant  traction  in  creating  HR videos  from LR  counterparts.  SR  quality  can  be  enhanced  by  aligning  features  extracted  from  videos  to  correct  motion  errors.  Existing  SR methods  often  suffer  from  challenges  such  as  misalignment  and  insufficient  temporal  information,  resulting  in  artifacts.  Zhu et al. [40] efficiently addressed the mentioned challenges based on the Deep VSR Network (DVSRNet). This approach integrates the Progressive Deformable Alignment (PDA) module and the Temporal-Sparse Enhancement (TSE) module. By propagating bidirectional information, the PDA module precisely aligns features and eliminates artifacts; furthermore, the TSE module enhances HR frames.

Chen et al.  [41] proposed a lightweight deep optical flow network to obtain bidirectional optical flows for the PDA module. A  new  loss  function  is  also  designed  to  generate  HR  frames  with  sharp,  precise  details.  The  results  presented  based  on  the demonstration  shows  that  the  method  is  superior  over  the  state-of-the-art  approaches.  This  approach  emphasizes  its  ability  to overcome  misalignment  issues  and  improve  the  overall  quality  of VSR.  Existing  approaches  based  on  deep  learning  methods commonly use LR optical flow for VSR, in which HR frames are generated from LR frames. However, LR optical flow may have difficulty recovering fine details. This work combines an optical flow SR and an optical flow enhancement algorithm to enhance temporal dependency accurately. A supplementary information component is extracted from adjacent LR frames for precise feature extraction. An optical flow enhancement algorithm is used to infer HR optical flow from LR. Motion compensation is then applied to the HR optical flows. As a second benefit, extracting the detail component mitigates the errors introduced by motion compensation based on optical flow. An SR network is then used to generate SR results. A comprehensive set of experiments on two datasets, Vid4 and DAVIS, demonstrate that the VSR method outperforms other state-of-the-art techniques to achieve high-quality SR (see Table 2).

### 2.3. Enhanced deep super-resolution (EDSR)

Enhanced  Deep  Super-Resolution  (EDSR)  is  a  cutting-edge  advancement  in  image  SR. The  EDSR  offers  an  improved way to reconstruct HR details from LR inputs. Lim et al.  [26] proposed EDSR architecture in 2017 as shown in   Fig. 3 and introduced key improvements to achieve optimal performance based on VDSR. By embracing a profound architecture, the EDSR can learn increasingly  complex  representations  of image  features,  surpassing  its  predecessors  in  the  number  of layers. As  a  result  of the incorporation of residual blocks, the optimization process is further simplified. The network can now learn the residuals between LR and HR images, resulting in highly detailed and realistic HR outputs from the model.

Table 2

Summary of VDSR Methods.

| Method | Key Features | Strengths | Limitations |
| --- | --- | --- | --- |
| VDSR [25] | Uses a deep CNN with | Achieves fine detail | Requires large training |
|  | residual learning to improve | reconstruction through deep | datasets and substantial |
|  | SR performance by learning | networks. Faster convergence | computational resources. |
|  | residuals between LR and HR images. | with depth-wise fine-tuning. |  |
| CVANet [33] | Utilizes cascaded visual | Outperforms state-of-the-art | Computationally intensive, |
|  | attention mechanisms | methods on PSNR, SSIM, | requiring high memory and |
|  | (feature, channel, and pixel attention modules) for image reconstruction. | and subjective perception. | processing power. |
| MSAN [34] | Combines multiscale self-attention and residual learning to enhance denoising performance for fluorescence and ultrasound images. | Achieves high PSNR for medical imaging denoising, particularly in real-time. | May not be generalizable to non-medical image denoising tasks. |
| FGVSR [39] | Uses temporal refinement | Outperforms existing VSR | Computationally demanding, |
|  | and alignment techniques for | methods with significant | especially for large video |
|  | improving VSR, preventing flickering and enhancing detail. | PSNR improvement. | datasets. |
| DVSRNet [40] | Integrates Progressive | Effective in overcoming | Requires sophisticated |
|  | Deformable Alignment and | misalignment issues and | optical flow algorithms and |
|  | Temporal-Sparse | improving VSR quality. | can be computationally |
|  | Enhancement modules to improve video SR. |  | expensive. |
| Optical Flow VSR [41] | Combines optical flow | Superior to conventional | Dependent on high-quality |
|  | enhancement with deep SR | methods in restoring fine | optical flow estimates; may |
|  | networks to generate | details and handling | struggle with complex |
|  | high-quality HR video frames. | temporal dependency. | motion patterns. |

Fig. 3.  EDSR Architecture.

With  its  streamlined  and  simplified  architecture,  EDSR  distinguishes  itself not  only  by  its  depth  but  also  by  its  simplicity. The  EDSR  has  consistently  achieved  top-tier  results  in  super-resolution  challenges  through  extensive  experiments  and  rigorous assessments. With its ability to capture fine details and produce visually appealing results, EDSR has become a benchmark in the field. The EDSR inspires researchers to explore the potential of increasingly sophisticated deep-learning architectures. EDSR influences subsequent developments in image super-resolution. By leveraging deep neural networks, EDSR demonstrates the solution to complex mappings that primarily improve images.

### 2.4. Super-resolution generative adversarial network (SRGAN)

A breakthrough in image super-resolution can be attributed to the Super-Resolution Generative Adversarial Network (SRGAN). The SRGAN incorporates the power of generative adversarial networks (GANs). Ledig et al. [27] introduced SRGAN in 2017 as a new paradigm incorporating adversarial training. SRGAN consists of a generator network that generates SR and a discriminator network that distinguishes between HR images. Using this adversarial training framework, the Generator produces HR outputs aligned with

ground truth images and perceptual quality. A more comprehensive evaluation of SR performance is possible with SRGAN. The SRGAN emphasizes the subjective visual realism of the generated images. It transcends traditional metrics such as PSNR and SSIM by emphasizing the subjective visual realism of the generated images.

In  the  Internet  of Things  (IoT)  context,  intelligent  cameras  are  increasingly  being  used  to  perform  quality  inspection  tasks. However,  low-light  conditions  may  pose  operational  challenges  for  these  smart-cameras.  This  situation  severely  degrades  the acquired  images, resulting  in  a  decline  in the performance  of the  subsequent  detection  algorithms.  Jing  et  al.  [42] proposed  a Low-light image Enhancement GAN (LEGAN) for industrial internet of smart-cameras model for the industrial internet of intelligent cameras, focusing on non-stationary noise compression and detail recovery. The input is decomposed into two branches, of which the first step uses Harr-wavelets. These branches are encoded independently using compact residual blocks, facilitating effective noise suppression. The second step incorporates a feature selection module to extract correlations between image foregrounds and backgrounds.

Further, the reconstruction is accomplished based on comprehensive feature map correlations. In the training phase, an optimal balance  between  noise  suppression  and  detail  recovery  is  ensured  by  an  adversarial  loss  comprising  MSE  loss,  VGG  loss,  and discriminating  loss.  It  is  evident  from  the  results  presented  that  the  LEGAN  performs better  than  other  noise  compression  and detail recovery schemes. In industrial practice, a high detection accuracy was achieved in low-light environments, specifically for yarn evenness inspection.

In SISR, an HR image is reconstructed from a LR image. This ill-posed problem arises because one LR image maps to multiple HR possibilities. It has been demonstrated that generative models-based SISR methods outperform traditional approaches in terms of performance. However, GANs, Variational Auto-Encoders (VAEs), and Flow face challenges such as unstable training, low sample quality, and high computational costs. As a result, these models need help to achieve the trifecta of diversity, high quality, and fast sampling. Despite their impressive sample variety and quality, denoising diffusion probabilistic models do not apply to real- world situations due to their prohibitive sampling costs. To overcome this limitation, Xiao et al. [43] proposed SISR with Denoising Diffusion GANs  (SRDDGAN), which aims for large step denoising, diverse samples, and stable training results. SRDGAN utilizes multimodal conditional GANs for each denoising step, combining denoising diffusion models with GANs. The results demonstrate that the SRDDGAN outperforms existing diffusion model-based methods regarding PSNR and perceptual quality metrics. The latent variable  Z  solution  significantly  improves  HR  spatial  domain  diversity.  In  addition,  SRDGAN  infers nearly  11 times  faster than diffusion-based SR3, making it more practical for real-world applications.

A retinal fundus image is an essential diagnostic tool in ophthalmology that facilitates the early detection and monitoring of various ocular diseases. It is challenging to capture HR fundus images due to factors such as defocusing, diffraction, limited shutter speed,  sensor unit  density,  and random noise.  Super-resolution  techniques  can  overcome this limitation, which  enhances visual details in retinal fundus images. The traditional SR methods tend to introduce artifacts into texture details and compromise delicate retinal vessel structures. Liquan et al. [44] proposed GANs that consist of a generator and a discriminator to improve the perceptual quality of retinal fundus images. A generator comprises 23 multiscale feature extraction blocks, an image segmentation network, and 23 residual-in-residual dense blocks. These components are strategically used to extract retinal vessel grayscale images and vascular features. It consists of two branches, one for extracting global features and another for extracting vascular features.

Further, the extracted features are fused to restore the SR image as best as possible. This approach can restore retinal vessel images in more detail and accurately. Additionally, attention modules are added to the Discriminator to improve the quality of SR  images. An  artifact loss  function  is  added to the  GAN to  enhance the  ability to measure the  disparity between the  HR  and restored images. Compared with state-of-the-art methods of image super-resolution, the images generated by this method exhibit better perceptual quality.

Guo et al.  [45] proposed a crucial aspect of maintaining high-quality spatial resolution in remote sensing images by preserving image  texture  during  super-resolution  reconstruction.  The  deep  learning  super-resolution  methods  based  on  GANs  successfully recover image texture. This study aims to improve the SR reconstruction of medium and LR remote sensing images using an enhanced GAN. Enhancements to the residual block and refinement of the model loss function are introduced in the network, which builds upon the SRGAN. The model loss function seeks to minimize the pixel distance between the reconstructed remote sensing and the original image. In addition, the network restores texture details based on semantic categories when perceptual loss is targeted. This method outperforms SRGAN over other networks in both subjective and objective assessments. Based on the results presented, this approach generates more realistic and reliable textures, improving the objective quantitative evaluation of the reconstructed images.	Kabiraj et al. [46] examine LR images of number plates where poor boundary contrast and accuracy make it challenging to detect numbers. To improve detection and recognition accuracy, low-quality number plate images must be digitized correctly, and image resolution needs to be enhanced. This approach is based on enhanced SRGAN, which modifies native dense blocks with a residual dense block model. An activation layer based on Rectified Linear Units (ReLUs) complements CNNs for threshold. Enhanced plate images are segmented using an optical character recognition (OCR) model to detect and recognize characters on the number plates.

The OCR model achieves an average accuracy of 84% for HR images, while for LR images, it varies from 4% to 7%. As the resolution of the plate images increases, the model accuracy increases, demonstrating its effectiveness. According to the OCR model, Enhanced Super-Resolution with Generative Adversarial Network (ESRGAN) provides superior LR image enhancement compared to SRGAN and Pro-SRGAN methods. Converting LR images to HR ones improves digit or alphabet detection accuracy on number plates. This approach improves number plate recognition reliability in LR situations.

Yu et al.  [47] address the challenges  of obtaining clear images of particleboard surfaces. This essential forest product saves forestry resources and promotes low-carbon development by reusing wood processing residues. The large size of particle boards poses difficulties in capturing detailed image features, resulting in blurred defect outlines. To enhance the quality of particleboard

Table 3

Summary of SRGAN Methods.

| Method | Key Features | Strengths | Limitations |
| --- | --- | --- | --- |
| SRGAN [27] | GAN-based architecture for high-quality image super-resolution. Uses adversarial training with generator and discriminator. | Realistic high-resolution images, improved perceptual quality. | Training instability, computationally intensive. |
| LEGAN [42] | Focuses on low-light image enhancement for smart cameras. Incorporates noise suppression and detail recovery. | Effective for industrial inspection, improves detection in low-light. | May not generalize to other use cases. |
| SRDDGAN [43] | Combines denoising diffusion models with GANs for stable and fast training. | Faster training, higher perceptual quality compared to diffusion models. | Computationally complex, struggles with extreme noise. |
| GAN for Retinal Fundus Images [44] | Uses generator-discriminator approach for improving perceptual quality, with multiscale extraction blocks and attention modules. | High-quality SR for medical images, especially vascular features. | Requires large datasets, may not generalize to other medical areas. |
| Enhanced SRGAN for Remote Sensing [45] | Enhances SRGAN with residual blocks and refined loss function for better texture preservation. | Superior texture restoration, higher evaluation scores. | May not perform well in dynamic environments. |
| ESRGAN for Number Plate Recognition [46] | Enhanced SRGAN with residual dense blocks for improving LR number plate images, integrates OCR for detection. | Increases accuracy of number plate recognition. | Limited to number plate recognition applications. |
| SRDAGAN for Particleboard Surface Images [47] | Optimized SRGAN using dense skip connections and CBAM for improving particleboard images. | Outperforms traditional methods in texture clarity. | Computationally demanding, not suited for low-resource environments. |
| Dual-Path GAN for Satellite Images [48] | Dual-path architecture combining feature mapping attention with deep convolution for satellite SR. | Enhanced clarity, feature extraction for satellite images. | Complex architecture, high computational requirements. |

surface images, the study employs SR reconstruction technology, focusing on improving the SRGAN performance. The presented enhancement method is named SR dense attention GAN (SRDAGAN). This method refines the SRGAN model by removing the batch normalization (BN) layer.

Further, the Convolutional Block Attention Module (CBAM) is optimized for constructing dense blocks. In addition, dense skip connections are incorporated into the dense block construction. It is clear from the results presented that the training, verification, and testing conducted on 52,400 image blocks with HR and LR in a 3:1:1 ratio produces better results. Comprehensive evaluation metrics, such as PSNR, SSIM, and Learned Perceptual Image Patch Similarity (LPIPS), demonstrate the superiority of SRDAGAN over traditional methods such as BICUBIC, SRGAN, and SWINIR. The reconstructed images have more evident textures and provide a realistic representation of limitless features, indicating a significant enhancement in the model performance. The study discovers the reconstruction effect on images with defects, emphasizing that SRDAGAN successfully achieves high-quality SR reconstruction of particleboard images.

Ren  et  al.  [48]  address the hardware limitations,  complex  electromagnetic noise,  and harsh natural  environment  challenges associated  with  the  quality  of  satellite  remote-sensing  images.  To  enhance  the  clarity  of  these  images,  the  study  proposes  a dual-path  adversarial  generation  network  algorithm  specifically  designed  for  satellite  remote  sensing  image  SR.  The  generator structure incorporates a dual-path convolution  operation, combining a feature mapping attention mechanism with an enhanced deep convolutional network. The design uses LR images to extract both important features and deep features. A reconstruction layer optimizes the SR process by fusing the extracted data. Improvements are made to the algorithm’s loss function and Discriminator to better balance the output image and the Discriminator. The fine-tuning aims to bring the SR image closer to human perception. As evident from the results presented compared to other methods, this algorithm performs significantly better on the UCAS-AOD datasets https://hyper.ai/datasets/5419 (see Table 3).

### 2.5. Image quality evaluation

It has become increasingly important to evaluate enhanced image quality as image super-resolution (SR) techniques continue to advance. To assess the performance of SR algorithms effectively, robust metrics must consider both objective and subjective aspects of image quality. Objective evaluation methods, such as PSNR, SSIM, and IFC [49] are widely used;

a.  Peak Signal to Noise Ratio

The PSNR is a widely used metric for evaluating image quality, particularly in the context of lossy image compression and reconstruction techniques such as super-resolution. The PSNR is expressed in decibels (dB) and is calculated as Eq. (1):

where L is the maximum possible pixel value of the image, and MSE is the Mean Squared Error between the original image and the reconstructed image, defined as Eq. (2):

In this formula I(i;j) represents the pixel value at position (i;j) in the original image. K(i;j) represents the pixel value at position (i;j) in the reconstructed image, m and n are the dimensions of the images. Combining these, the PSNR formula can be written as shown in Eq. (3):

PSNR = 10 . log                                                             (3)

b.  Structural Similarity Index Measurement (SSIM) The next metric used to assess the SR methods is the SSIM. The SSIM is a method for measuring the similarity between two images. It considers changes in structural information, luminance, and contrast to assess image quality. SSIM is widely used because it provides a more comprehensive evaluation of image quality compared to metrics such as MSE and PSNR. The SSIM index between two images x and y is defined as Eq. (4):

where   x  is the average of image x,   y  is the average of image y,    represent the variance of image x,    is the variance of

image y,   xy  is the covariance of images x and y, C1 is given by (K1L)2 and C2 is given by (K2L)2

In the above equations L is the dynamic range of the pixel values (e.g., 255 for 8-bit grayscale images). K1 and K2 are small constants with the typical values, K1 = 0:01 and K2 = 0:03.

c.  Information Fidelity Criterion (IFC) The IFC metric is used to assess the quality of an image by quantifying the amount of information shared between a reference (original) image and a distorted (reconstructed) image. It is based on the concept of natural scene statistics and the notion that the human visual system is highly adapted to the statistical properties of natural scenes. The IFC is derived from the mutual  information between the reference image and the distorted  image. It can be expressed as shown in Eq. (5):

where I is the reference image, K is the distorted image, xj  and yj  are the coefficients of the reference and distorted images,

respectively,  in  a multi-scale, multi-orientation  image  decomposition  (e.g., wavelet transform),   j    is  the variance  of the

reference image coefficients,   j   is the variance of the distorted image coefficients,    jyj   is the covariance of the reference

and distorted image coefficients, and N is the number of coefficients.

Despite of objective evaluation metrics presented, there remains a gap in objective evaluation criteria that align closely with human subjective perception. In the following section, the Perceptual Index (PI) and Root Mean Square Error (RMSE) metrics that are based on human perception of image quality are presented.

d.  Perceptual Index (PI) The PI is an image quality metric that combines the Ma score and the Naturalness Image Quality Evaluator  (NIQE)  to  approximate human perception  of image  quality. This  approach  is  especially valuable  in  evaluating images that have been enhanced or generated, as it accounts for both aesthetic appeal and naturalness. The Perceptual Index is calculated using Eq. (6)

where the Ma score measures the aesthetic quality of the image, with higher scores indicating greater aesthetic appeal, and NIQE evaluates the naturalness of the image, where lower NIQE scores suggest a more natural appearance. Together, these scores provide a balanced assessment of how visually pleasing an image may be to a human observer.

A lower Perceptual Index score suggests a higher perceived image quality, reflecting both a high Ma score (indicating visually pleasing or attractive to human observers) and a low NIQE score (indicates naturalness). Conversely, a higher PI score typically indicates a less visually satisfying image. This metric is particularly useful in image enhancement tasks such as low-light image enhancement, super-resolution, and de-noising, where human perception of quality is essential. In applications like GANs for image synthesis, the Perceptual Index can help assess the realism of generated images. Moreover, it is helpful for comparing different image enhancement algorithms, providing a perceptually relevant comparison that aligns with human satisfaction.

e.  Root Mean Square Error (RMSE) RMSE is a commonly used metric to evaluate the performance of regression models in machine learning. It quantifies the average magnitude of errors between predicted values and actual (observed) values. RMSE is calculated by first finding the difference between each predicted value and its corresponding actual value, then squaring these differences to remove negative signs and give more weight to larger errors. These squared errors are averaged, and then the square root of this average is taken, yielding the RMSE value. Mathematically, RMSE is given by Eq. (7):

Root Mean Square Error, RMSE = Ⅴ                                                                                                      (7)

where yi  represents the actual values, i  represents the predicted values, and n is the total number of observations.

RMSE is particularly useful in regression since it is sensitive to larger errors and provides a metric in the same units as the target variable, making it easy to interpret. In machine learning, RMSE is commonly used for model evaluation: a lower RMSE indicates a model that better fits the data. RMSE also aids in model comparison, where the model with the lowest RMSE is typically considered the best. Furthermore, in hyperparameter tuning, RMSE often serves as the metric to minimize when optimizing model parameters, such as through grid or random search, to achieve better accuracy.

The evaluation schemes offer a comprehensive understanding of the effectiveness of image SR techniques in enhancing visual appeal and perceptual accuracy.

## 3.  Proposed method

It is clear from the introduction section that low-light image and video enhancement present complex challenges beyond simple brightness adjustment. In low-light environments, issues such as artifacts, color distortion and noise are common. Techniques are needed to ensure accurate color representation, remove artifacts, and to apply appropriate filters for noise reduction. Increasing brightness in such settings is also challenging due to noise amplification. Therefore, methods that reduce noise while preserving visual clarity are crucial. Additionally, fine details can be lost due to independent image processing, making it necessary to balance global enhancements for improved visibility with the preservation of local details. Achieving this balance requires addressing both subjective  and  objective performance metrics,  considering  diverse  scenes,  and  adapting to real-time processing  constraints. The integration  of enhancement  techniques,  user  preferences,  and  metrics  based  validation  further  complicates  the  development  of effective low-light image and video enhancement methods.

The proposed method incorporates an adaptive strategy, advanced algorithms, and comprehensive assessment metrics to address the identified challenges. The framework, Dual SRGAN, adaptively fuses the MRSRGAN and WPSRGAN networks.  Fig. 4 presents the flow diagram of the proposed architecture. The perception scores  [50] and RMSE are used to achieve an optimal balance between global enhancement and local detail preservation. The MRSRGAN improves objective performance by enhancing fine details, while subjective performance is improved by the WPSRGAN. A thresholding mechanism is used to adaptively fuse the MRSRGAN and WPSRGAN networks.

Additionally, a top-hat gradient-domain filter is employed to remove noise and artifacts resulting from SR. PSNR, SSIM, and IFC metrics are used to validate the results of the proposed method. The experimental results presented demonstrate the effectiveness of the proposed approach. As shown in the results, the proposed method quantitatively outperforms current state-of-the-art techniques. Under low-light conditions, it can produce vivid, clear, and brilliant images and videos that closely resemble human visual system perception.

The Sections 3.1, 3.2, and 3.3 introduces the architectures of the MRSRGAN and WPSRGAN networks, followed by their adaptive fusion process. Further, the overall architecture of the Dual SRGAN framework is presented.

### 3.1. Architecture of MRSRGAN

MRSRGAN enhances fine details in images and videos by utilizing Memory Residual (MR) blocks. The memory residual blocks store and leverage information from previous layers to emphasize and refine the complex features. Through training, the network improves its ability to capture and highlight delicate structures under low-light conditions by learning and retaining critical details.

Fig. 5 presents the complex architecture of MRSRGAN, which is explicitly designed for SR tasks. A single convolutional layer extracts  features  from  LR  images,  serving  as  input  for  both  the  Generator  and  Discriminator  of the  MRSRGAN.  Image  quality is  enhanced  through  the  integration  of seven  MR  blocks  to  the  architecture  of MRSRGAN.  The  integrated  MR  blocks  improve the model’s ability to retain important features by creating persistent memory. By incorporating residual connections, the model effectively captures long-range dependencies, emphasizing the differences between LR and HR features. The Generator refines these features iteratively using residual blocks, followed by up-sampling blocks. These up-sampling blocks increases the spatial resolution of images  and videos.  Finally,  the  HR  images  are  generated  through  the  last  convolutional  layer,  demonstrating  the  network’s capability.

Fig. 4.  (a) Flow  Diagram of the Proposed Architecture  (b) Image Transformation of Adaptive Dual SRGAN.

A sophisticated mechanism unfolds within each MR block. Each MR block consists of four Residual Blocks (ResBlocks) and a gate unit. After generating these groups, the gate unit extracts a specified number of features. Combining the input features with the extracted features yields the final output of the MR block. The ResBlocks are transformed using Parametric Rectified Linear Unit (PReLU) functions, which replace the activation function layers. To simplify the overall architecture, the BN layers are excluded from the computation. Two up-sampling operations are necessary to restore the original image size. Each convolutional layer is assigned a stride, with parameters represented as n indicating the number of corresponding feature maps. This configuration optimizes the generator network’s computational efficiency while ensuring that the original image’s essential structural and functional aspects are preserved during SR processing. The discriminator network employs the same settings as the generator network.

#### 3.1.1. Loss function

The total generator loss function, as defined by Eq.  (8), can be decomposed into pixel-wise, adversarial, and perceptual loss components.

Total Loss, Ltotal  = Lpixel + 1Ladv + 2Lvgg                                                                                                                                                                                                           (8)

In the defined loss formulation, three distinct components contribute to the overall objective function for the MRSRGAN: pixel- wise Mean Squared Error (MSE) loss (Lpixel), perceptual loss based on the VGG19 network features (Lvgg), and the adversarial loss for the Generator (Lad v). Each term serves a specific purpose in guiding the training of the generator network to produce high-quality SR images.

a.  Pixel-wise MSE Loss (Lpixel): The Lpixel  represented by Eq.  (9) quantifies the pixel-wise mean squared error between the generated images and the ground truth. Here,  G(xl) denotes the Generator that produces the SR images, while xl  and xt represents the LR input images and their corresponding ground truth, respectively. This loss term focuses on minimizing the discrepancy at the pixel level, aiding the Generator in producing images that closely resemble the ground truth in terms of raw pixel values.

Pixel-wise MSE Loss, Lpixel                                                                                                                                                                               (9)

(b) Discriminator

Fig. 5.  The Memory Residual SRGAN Architecture  (a) Generator,  (b) Discriminator.

b.  Perceptual Loss (Lvgg): The Lvgg  loss defined by Eq.  (10) introduces perceptual loss, calculated as the MSE loss between features extracted from the pre-trained VGG19 network. By utilizing features from a deep CNN like VGG19, the Generator is driven to produce images that not only match the ground truth at the pixel level but also capture higher-level semantic information. This enhances the perceptual quality of the generated images. The number of training samples is indicated as

N. The term    represents the features extracted from the pre-trained VGG19 network.

c.  Adversarial Loss (Lad v): The Lad v  loss, defined by Eq. (11) represents the adversarial loss for the Generator, a fundamental component of the GAN framework. In this context, the logarithm is omitted from the adversarial loss calculation. Adversarial loss helps the Generator to produce images that appear homogeneous when evaluated by the Discriminator. The weights associated  with  adversarial  and  perceptual  losses  are  denoted  by    1 and     2,  respectively.  The  combined  loss  function integrates pixel-wise accuracy, perceptual quality, and adversarial training to guide the MRSRGAN in generating high-fidelity super-resolved images. The weights    1 and    2 allow for fine-tuning the contributions of adversarial and perceptual losses during the training process.

Adversarial Loss, Ladv  = _D(G(xl ))                                                                          (11)

### 3.2.  WPSRGAN architecture

WPSRGAN improves overall image enhancement by incorporating a weighted perceptual loss. This approach enhances subjective performance by prioritizing overall visual quality and global features. The network assigns different levels of importance to various perceptual features, ensuring a balanced and enhanced global appearance.

The WPSRGAN architecture, comprising 16 ResBlocks within the generator network, is shown in  Fig. 6. In the Generator, each ResBlock  consists  of a  convolutional  layer,  a PReLU  activation  layer,  and  a  second  convolutional  layer. WPSRGAN  deliberately omits BN layers from both the generator and discriminator components. The detailed structure of each ResBlock in the Generator is outlined as follows:

Fig. 6.  The Generator Architecture of WPSRGAN.

Convolutional Layer (First):The ResBlock begins with a convolutional layer that extracts features from the input data. This layer performs spatial convolution to capture essential information relevant to the super-resolution task.

PReLU Activation  Layer:  The  convolutional  layer  is  followed  by  a  PReLU  activation  layer.  PReLU  provides  a  non-linear activation function, enabling the network to model complex relationships and capture complex details in the data.

Convolutional  Layer  (Second): A  second  convolutional  layer  refines  the  features  extracted  by  the  preceding  layers.  This dual-convolutional structure within each ResBlock allows for more effective capture and representation of hierarchical features.

The decision to remove BN layers from both the Generator and Discriminator ofWPSRGAN was made to simplify the architecture and reduce computational complexity. BN layers can sometimes introduce unwanted artifacts or hinder training stability, which led to their exclusion. The Discriminator in WPSRGAN is similar to that of MRSRGAN, with the key distinction being the absence of BN layers. The Discriminator distinguishes between authentic and generated images, and this architectural adjustment maintains consistency between the two models while emphasizing the removal of BN layers.

The WPSRGAN network architecture consists of 16 ResBlocks in the Generator, each comprising a convolutional layer, a PReLU activation layer, and another convolutional layer. The absence of BN layers in both the Generator and Discriminator aligns with the weighted perceptual loss approach and underscores the emphasis on simplicity in design.

Loss Function: As depicted in   Fig. 6 , WPSRGAN employs a two-stage bias adversarial training mechanism utilizing distinct generator losses. During the initial stage, highlighted, the generator loss is optimized, comprising pixel-wise loss and adversarial loss, to enhance the overall objective performance (specifically, to minimize the RMSE value). In the subsequent stage, the network parameters  from  the  first  stage  is  treated  as  a pre-trained model.  Subsequently,  generator  loss  is replaced with perceptual  and adversarial loss to refine and optimize subjective visual effects (e.g., minimizing the perceptual index). The two-stage losses are defined by Eqs. (12) and (13).

L1 = Lpixel+ λ 1Ladv                                                                                                                                                                                                                                                                    (12)

L2 = λ 1Ladv+ λ2Lvgg                                                                                                                                                                                                                                                                 (13)

Here, the pixel-wise loss is defined as Eq. (9), the perceptual loss utilizes MSE loss calculated based on features extracted from a pre-trained VGG19 network, and the adversarial loss is represented as Eq. (14),

Ladv  = − log(D(G(xl )))                                                                                           (14)

Implementing a two-stage adversarial training mechanism facilitates aligning the generated SR image with the corresponding ground truth in the high-level features space.

### 3.3. Adaptive fusion

The adaptive fusion approach employed in the proposed framework maximizes the strength of both networks by combining MRSRGAN and WPSRGAN outputs. To align the perceptual score with RMSE in SR results, a soft thresholding method is applied.

The network outputs are then merged using a dynamic mechanism, where the threshold is adjusted based on specific characteristics of the image or video inputs.

A distinct SR outcomes are derived using the two previously mentioned GANs. One is MRSRGAN, prioritizing enhanced objective performance. The other is WPSRGAN, which achieves results with superior subjective perception. To reunite the perceptual score, the  soft  thresholding  method  proposed by  Deng  et  al.  [13]  is  adopted.  In  order  to merge  the  two  SR  images  (i.e.,  MRSRGAN, WPSRGAN), a technique similar to a form of pixel interpolation is adopted. Eq. (15) presents the soft thresholding method adopted in the proposed work:

Ie = IG + soft( ;  )                                                                                                (15)

where Ie  is the fused image and    =(IG-Ig ), IG  is the generated image by WPSRGAN whose perceptual score is lower, Ig  is the generated image by MRSRGAN whose RMSE value is lower.    is the adjusted threshold. The soft( ;  ) is defined in Eq. (16)

soft( ;  ) = sign( ) . max(|   | − ; 0)                                                                                  (16)

### 3.4. Dual SRGAN (DSRGAN) framework

The adaptive fusion approach presented in the earlier section integrates MRSRGAN and WPSRGAN networks to arrive at the Dual SRGAN (DSRGAN) network. This approach leverages the specific strengths of each network to enhance low-light images and videos comprehensively. A DSRGAN is based on the fusion of MRSRGAN and WPSRGAN results. DSRGAN represents a unified network that integrates the enhanced fine details of MRSRGAN and the globally improved perceptual quality of WPSRGAN. Integrating local and global aspects of low-light content is crucial.

### 3.5.  Top-hat gradient domain filtering (THGDF)

Dual  SRGAN  outputs  are  subjected  to top-hat  gradient-domain  filtering  to remove  image  artifacts, noise  and balance  global enhancement  and  local  detail  preservation.  The  filtering  process  refines  the  enhanced  results  to  effectively  address  low-light conditions and other challenges while maintaining high visual fidelity.

#### 3.5.1.  Top-hat transformation

The top-hat transformation [51] is the most widely used operation in mathematical morphology. It is used for image enhancement which involves adding bright areas while subtracting dark areas from the original image. Enhancing the performance of the top-hat transform is often employed within a multiscale framework. This approach allows the multiscale top-hat transform to extract various beneficial features from the image, subsequently utilized for infrared image enhancement. Widely adopted across diverse grayscale image  types,  the  multiscale  top-hat  transform  scheme has  demonstrated  efficacy  in  enhancing  retinal,  ultrasound,  and  infrared images. Furthermore, it finds application in tasks such as visible and infrared image fusion, image segmentation, and small object detection.

The  top-hat  transform  constitutes  a  composite  operation  within  mathematical  morphology  derived  from  other  fundamental morphological operations, including erosion, dilation, opening, and closing. The morphological operations of dilation and erosion applied to I(u,v) for B(s,t), denoted as I 田 B and I θ B are defined as shown in Eqs. (17) and (18):

(I 田 B)(u; v) = (I {I(u + s; v + t) + B(s; t)}                                                                          (17)

(I θ B)(u; v) = ()B {I(u + s; v + t) + B(s; t)}                                                                          (18)

where,  I  represents  the  original  input  image,  with  pixels  corresponding  to  all  spatial  coordinates  (u,v),  while  B  denotes  the structuring element, represented by spatial coordinates (s, t). The morphological operations of opening I θ B and closing I 田 B on I(u; v) for B(s,t)are derived from the dilation and erosion operations as shown in Eqs. (19) and (20):

I ◦B = (I θ B) 田 B                                                                                                                                                                  (19)

I . B = (I 田 B) θ B                                                                                                                                                                  (20)

The morphological operation of the top-hat transform  [51] is constructed from morphological opening and closing. White Top- Hat (WTH) represents the top-hat transform via opening, while Black Top-Hat (BTH) characterizes the top-hat transform via closing. WTH captures bright areas, whereas BTH captures dark areas omitted in the opening and closing operations. Both transformations are defined as shown in Eqs. (21) and (22):

WTH = I − (I◦B) = I − ((I θ B) 田 B)                                                                              (21)

BTH = (I . B) − I = ((I 田 B) θ B) − I                                                                                                                                     (22)

#### 3.5.2. Modified top hat transform

The classical top-hat transform typically relies on a single structuring element, resulting in inefficiencies in image processing [52]. To enhance the performance of the top-hat transform, it is suggested to employ two structuring elements, proportional geometry, and flatness [53]. The Modified White Top-Hat (MWTH) and the Modified Black Top-Hat (MBTH) transforms are utilized for image enhancement within the framework of multiscale top-hat transform. Let G and G’ represent the structuring elements with geometric proportionality and flatness. Then, the top-hat transform employed in the multiscale framework is defined as shown in Eqs. (23)

and (24):

MWTH = I _ ((I θ G) 田 G9)                                                                                       (23)

MBTH = ((I 田 G) θ G9) _ I                                                                                                                                                     (24)

#### 3.5.3. Proposed multiscale top-hat transform

The method proposed here centers on the multiscale top-hat transform, leveraging two structuring elements within the transform to enhance its effectiveness. This approach enhances the image details, improves contrast, and preserves mean brightness. Initially, the image enhancement algorithm utilizes the following parameters: the original image I, the number of iterations n within a range iε1,2,. . . n,n> 1 and two structuring elements G and G’ Multiple Brightness (MB) and Multiple Darkness (MD) regions will be derived through the top-hat transform in the following Eqs. (25) and (26):

MBi = I _ ((I θ Gi) 田 Gi9)                                                                                          (25)

where MBi  is the i-scales of brightness extracted from the image, and Gi  and G9i will grow in each iteration.

MDi = ((I 田 Gi) θ Gi9) _ I                                                                                                                                                        (26)

where MDi  are the i-scales of darkness extracted from the image.

## 4.  Experimental evaluation and discussion

This section outlines the experimental setup, parameter settings, benchmark methods, datasets, evaluation metrics used to assess the  performance  of the  proposed  method.  It  also  includes  a  performance  evaluation  and  a  comparative  analysis  with  existing methods.

### 4.1. Experimental setup

The experiment was conducted on a workstation running Windows 11. The setup included a PC with an Intel(R) Core(TM) i9- 10940X CPU @ 3.30 GHz, an NVIDIA GeForce RTX 3080 GPU, and 64 GB of RAM with CUDA 12.2 and CUDNN 8.9. The development environment was JetBrains PyCharm 2021.2.3 Professional edition, and Python was used to implement the relevant algorithms.

### 4.2. Parameter settings

This section presents the parameters used in the proposed method, with their values detailed in Table 4.

### 4.3. Benchmark methods

The  benchmark  methods  that  are  used  to  validate  the  proposed  method  are  BGAN  [16],  RGSR  [17],  BDRVSR  [18],  RESR- GAN  [19], DBVSR  [20], and BlindSR  [21]. The results of the proposed method is compared to that of reference methods to assess the performance.

### 4.4. Datasets

In the proposed work, images and videos are sourced from five publicly available benchmarks for 4× image SR: Sun-Hays [54], Urban100  [55], Set5  [56], Set14  [57], and a mobile dataset. The Sun-Hays, Urban100, Set5, and Set14 datasets primarily contain natural images, while the mobile dataset includes images captured with various devices: iPhone 3GS, BlackBerry Passport, Sony Xperia Z, and Canon 70D DSLR. The mobile dataset is freely accessible to the public and can be downloaded at http://people.ee. ethz.ch/~ihnatova/#dataset.

The Sun-Hays dataset is a important benchmark in image SR, featuring a diverse range of natural and urban scenes that provide HR and corresponding LR image pairs. This dataset’s complexity and variety make it an essential resource for training SR models and evaluating their performance in reconstructing detailed HR images from LR inputs. The Sun-Hays dataset’s comprehensive nature makes it highly relevant for real-world applications such as satellite imaging, urban planning, and autonomous driving.

The Set5 dataset is a widely used benchmark in image SR research, consisting of five high-quality images featuring diverse scenes, including natural and urban settings. These images are accompanied by their corresponding LR versions, created through standard

Table 4

Parameter Settings.

| Setting | Value |
| --- | --- |
| Dataset for Training | Flickr2K (2650 HR images) |
| Dataset for Evaluation | Urban100, Sun-Hays, Set5, Set14 (100+ images) |
| Image Preprocessing | Bicubic downsampling for LR images |
| LR Patch Size | 24 × 24 |
| HR Patch Size | 96 × 96 |
| Data Augmentation | Random horizontal and vertical flipping |
| Batch Size | 16 |
| Generator (MRSRGAN) | 7 multi-scale residual (MR) blocks |
| Filter Size (Generator) | 3 × 3 |
| Learning Rate (Generator) | 1 × 10−4, Adam optimizer with momentum of 0.9 |
| Training Epochs (Generator) | 500 |
| Generator (WPSRGAN) | 16 residual blocks |
| Filter Sizes (WPSRGAN) | Intermediate layers: 3 × 3, First & Last layers: 9 × 9 |
| Activation Function | Parametric ReLU (PReLU) |
| Learning Rate Schedule | Initial: 1 × 10−4, reduced by 10 after 250,000 iterations |
| Total Iterations (WPSRGAN) | 500,000 |
| Discriminator Filter Size | 3 × 3 |
| Discriminator Features | Starts at 64, doubles progressively up to 512 |
| Adversarial & Perceptual Loss Weights | Λ1 = 1 × 10−3, Λ2 = 6 × 10−3 |
| Image Fusion Threshold (ξ) | 0.73 |
| Performance Metrics | PSNR, SSIM, IFC, Perceptual Score, RMSE |
| Selected Model for Evaluation | WPSRGAN (two-stage) |
| Thresholding Range for RMSE | RMSE between 11.5 and 16 (Regions defined) |

down-sampling techniques. Set5 is commonly used for evaluating and comparing the performance of SR algorithms, focusing on their ability to enhance image quality and preserve details. Urban datasets widely used in SR methods which consist of 100 HR and corresponding LR images of distinct urban environments, including residential, commercial, and industrial areas. These datasets are important for training and evaluating SR algorithms, enabling the reconstruction of detailed HR images from degraded LR inputs. The Set14 dataset is a popular benchmark in the field of image SR, containing 14 diverse images that include both natural scenes and urban environments. Each image is provided in HR form, along with corresponding LR versions created through down-sampling techniques. This datasets is widely used to evaluate and compare the effectiveness of SR algorithms, particularly in their ability to reconstruct fine details and enhance overall image quality.

#### 4.4.1. Pre-processing for dataset quality enhancement

Preprocessing steps such as denoising, sharpening, and artifact reduction are crucial for enhancing the quality and consistency of datasets used in machine learning, particularly for image and video enhancement tasks. Denoising techniques, such as Gaussian filtering or Non-Local Means (NLM), assists to eliminate noise, ensuring the model focuses on meaningful features. The sharpening methods, such as unsharp masking, emphasize fine details and edges, which are essential for tasks like super-resolution. Artifact reduction techniques,  including Total Variation  (TV) minimization,  is used to  eliminate  compression  artifacts that  could  distort image content. These preprocessing steps improve model performance by ensuring high-quality, relevant data, and contribute to the study’s reproducibility and robustness by providing a clear and replicable pipeline for future research.

a.  Image Denoising Techniques: The  Gaussian  filtering  is  applied  on  the  datasets  to  reduce  random  noise  in  the  images while maintaining the integrity of important image structures. Additionally, the NLM denoising was used to address spatially correlated noise, particularly effective for repetitive patterns within images.

b.  Sharpening Approaches: Unsharp masking was implemented to enhance image edges and fine details by amplifying high- frequency components. Furthermore, high-pass filtering was utilized to emphasize edges and textures, which are critical for the fidelity of super-resolved outputs.

c.  Artifact  Reduction:  To  mitigate  compression  artifacts  (i.e.  blocking  and  ringing  artifacts  commonly  found  in  JPEG- compressed  images),  the  TV  is  used  as  a  minimization  algorithm.  Additionally,  wavelet-based  artifact  correction  was performed to reduce artifacts while preserving image features.

These preprocessing steps were applied consistently across the dataset to ensure uniformity and enhance the quality of the input data. By refining the input data through denoising, sharpening, and artifact reduction, the training process was better aligned with the high-quality outputs expected from the super-resolution algorithm.

The  Figs. 7 to 9 shows the first set of experimental results from Sun-Hays dataset. The Fig. 7(a) shows the ground truth image with a pixel resolution of 1200 × 768. The standard bicubic interpolation method is applied to the LR image. The bicubic interpolation produces smoother and higher quality images compared to bilinear interpolation. It preserves edges and details in the reconstructed image. However, as is evident from the cropped bicubic interpolated image presented in   Fig. 7(b), image blur and artifacts still exist.   Fig. 7(c) shows the result proposed by Luo et al.  [16]. The results obtained from a two-step lossy JPG image SR based on noise reduction proposed by Biao et al.  [17] are shown in  Fig. 7(d).  Fig. 7(e) shows the SR result proposed by Wang et al.  [18].

Fig. 7.  Visual Comparison of 4× Reconstructed SR Horses image from Sun-Hays Dataset: (a) High-resolution (PSNR, SSIM, IFC) (b) Bicubic (27.09, 0.76, 1.48) (c) BGAN  [16] (27.89, 0.72,  1.75), (d) RGSR  [17] (28.05, 0.75,  1.77), (e) BDRVSR  [18] (27.89, 0.73,  1.73), (f) RESRGAN  [19](27.38, 0.72,  1.50), (g) DBVSR  [20] (27.98, 0.78,  1.71),  (h) BlindSR  [21]  (28.17, 0.74, 2.06), and  (i) Proposed Method  (28.19, 0.79,  1.97).

Fig. 8.  Visual Comparison of 4× Reconstructed SR Oil Container image from Sun-Hays Dataset: (a) High-resolution (PSNR, SSIM, IFC) (b) Bicubic (25.11, 0.74, 1.8)  (c)  BGAN  [16]  (25.47,  0.76,  1.99),  (d)  RGSR  [17]  (25.56,  0.77,  2.02),  (e)  BDRVSR  [18]  (25.36,  0.76,  2.39),  (f)  RESRGAN  [19](25.21,  0.75,  1.74),  (g) DBVSR  [20]  (25.64, 0.71, 2.04),  (h) BlindSR  [21]  (25.64, 0.77, 2.37), and  (i) Proposed Method  (25.69, 0.78, 2.38).

Fig. 9.  Visual  Comparison  of 4× Reconstructed  SR  McDonald  image  from  Sun-Hays  Dataset:  (a)  High-resolution  (PSNR,  SSIM,  IFC)  (b)  Bicubic  (29.22,  0.79, 2.5)  (c)  BGAN  [16]  (29.95,  0.80,  2.83),  (d)  RGSR  [17]  (29.98,  0.75,  2.81),  (e)  BDRVSR  [18]  (29.96,  0.71,  2.80),  (f)  RESRGAN  [19](29.45,  0.79,  2.24),  (g) DBVSR  [20]  (29.92, 0.81, 2.82),  (h) BlindSR  [21]  (30.08, 0.77, 3.09), and  (i) Proposed Method  (30.09, 0.80, 3.18).

The Real-ESRGAN result by Xintao et al. [19] is shown in  Fig. 7(f). Deep Blind Video Super-resolution result is shown in  Fig. 7(g). Blind image Super-resolution result is shown in  Fig. 7(h).  Fig. 7(i) shows SR results of the proposed method.

The   Figs. 10 to 12 shows the second set of results of Urban dataset.   Figs. 13 through 15 shows the third set of results from Set14 dataset. In this dataset, we have selected five sample images out of 14 images.

The Tables 5 through 13 shows the PSNR, SSIM and IFC values for the visual comparisons results presented in  Figs. 7 to 15.

Three distinct components of the defined loss formulation contribute to the overall objective function of the MRSRGAN, namely, the pixel-wise mean squared error (MSE) loss, the perceptual loss based on the VGG19 network features and the adversarial loss for the Generator network. Each term serves a specific purpose in guiding the training of the generator network to produce high-quality SR images.

(a)                                                             (b)                                                             (c)                                                             (d)

(e)                                                             (f)                                                             (g)                                                             (h)

Fig. 10.  Visual Comparison of 4× Station image from Urban Dataset: (a) HR, (b) BGAN  [16], (c) RGSR [17], (d) BDRVSR [18], (e) RESRGAN [19], (f) DBVSR [20], (g) BlindSR  [21], and  (h) Proposed Method.

(a)                                                             (b)                                                             (c)                                                             (d)

(e)                                                             (f)                                                             (g)                                                             (h)

Fig. 11.  Visual Comparison of 4× City image from Urban Dataset: (a) HR, (b) BGAN  [16], (c) RGSR  [17], (d) BDRVSR  [18], (e) RESRGAN  [19], (f) DBVSR  [20], (g) BlindSR  [21], and  (h) Proposed Method.

Table 5

Quantitative  Comparison  of SR Algorithms  using  PSNR values  on  Sun-Hays  Dataset  for  a  scaling  factor  4.  Red

color mark indicates the best results.

| Methods/Images | Scale | Horses | Oil Container | McDonald |
| --- | --- | --- | --- | --- |
| Bicubic | ×4 | 27.09↓ | 25.11↓ | 29.22↓ |
| BGAN [16] | ×4 | 27.89 | 25.47 | 29.95 |
| RGSR [17] | ×4 | 28.05 | 25.56 | 29.98 |
| BDRVSR [18] | ×4 | 27.89 | 25.36 | 29.96 |
| RESRGAN [19] | ×4 | 27.38 | 25.21 | 29.45 |
| DBVSR [20] | ×4 | 27.98 | 25.64 | 29.92 |
| BlindSR [21] | ×4 | 28.17 | 25.64 | 30.08 |
| Proposed method | ×4 | 28.19↑ | 25.69↑ | 30.09↑ |

The network is trained using the Flickr2K dataset comprising 2650 HR images. HR images are down-sampled using the bicubic interpolation method to obtain LR images. The proposed models are assessed using the urban100, Sun-hays, Set5, and Set14 self- validation dataset, which includes more than 100 realistic images. For training, LR and HR image patches are randomly cropped from the corresponding LR and HR image pairs. The crop size for LR patches is set to 24 × 24, while the corresponding HR patches

(a)                                                             (b)                                                             (c)                                                             (d)

(e)                                                             (f)                                                             (g)                                                             (h)

Fig. 12.  Visual Comparison of 4× Glass image from Urban Dataset: (a) HR, (b) BGAN  [16], (c) RGSR  [17], (d) BDRVSR  [18], (e) RESRGAN  [19], (f) DBVSR  [20], (g) BlindSR  [21], and  (h) Proposed Method.

(a)                                                             (b)                                                             (c)                                                             (d)

(e)                                                             (f)                                                             (g)                                                             (h)

Fig. 13.  Visual Comparison of 4× Zebra image from Set14 Dataset: (a) HR, (b) BGAN  [16], (c) RGSR  [17], (d) BDRVSR  [18], (e) RESRGAN  [19], (f) DBVSR  [20], (g) BlindSR  [21], and  (h) Proposed Method.

Table 6

Quantitative  Comparison  of SR  Algorithms  using  SSIM  values  on  Sun-Hays  Dataset  for  a  scaling  factor  4.  Red

color mark indicates the best results.

| Methods/Images | Scale | Horses | Oil Container | McDonald |
| --- | --- | --- | --- | --- |
| Bicubic | ×4 | 0.76 | 0.74 | 0.79 |
| BGAN [16] | ×4 | 0.72↓ | 0.76 | 0.80 |
| RGSR [17] | ×4 | 0.75 | 0.77 | 0.75 |
| BDRVSR [18] | ×4 | 0.73 | 0.76 | 0.71↓ |
| RESRGAN [19] | ×4 | 0.72↓ | 0.75 | 0.79 |
| DBVSR [20] | ×4 | 0.78 | 0.71↓ | 0.81↑ |
| BlindSR [21] | ×4 | 0.74 | 0.77 | 0.77 |
| Proposed method | ×4 | 0.79↑ | 0.78↑ | 0.80 |

are cropped to 96 × 96. To augment the dataset, random horizontal and vertical flipping is applied to the image patches. The batch size is set to 16. In the Generator of MRSRGAN, seven MR blocks are used. The filter size is set to 3 × 3 for all convolutional layers. The learning rate is initialized at 1 × 10_4 and The Adam optimizer is used with a momentum value of 0.9. The network is trained for 500 epochs, and the best results are selected based on the PSNR, SSIM, and IFC metrics.

For the Generator of WPSRGAN, 16 ResBlocks are employed. The filter size for the intermediate layers is 3 × 3, while the first and last convolutional layers use a filter size of 9 × 9 . All convolutional layers have a stride of 1 and padding of 1. The weights

(a)                                                             (b)                                                             (c)                                                             (d)

(e)                                                             (f)                                                             (g)                                                             (h)

Fig. 14.  Visual Comparison of 4× Girl image from Set14 Dataset: (a) HR, (b) BGAN  [16], (c) RGSR  [17], (d) BDRVSR  [18], (e) RESRGAN  [19], (f) DBVSR  [20], (g) BlindSR  [21], and  (h) Proposed Method.

(a)                                                             (b)                                                             (c)                                                             (d)

(e)                                                             (f)                                                             (g)                                                             (h)

Fig. 15.  Visual Comparison of 4× Baboon image from Set14 Dataset: (a) HR, (b) BGAN  [16], (c) RGSR [17], (d) BDRVSR [18], (e) RESRGAN [19], (f) DBVSR [20], (g) BlindSR  [21], and  (h) Proposed Method.

Table 7

Quantitative Comparison of SR Algorithms using IFC values on Sun-Hays Dataset for a scaling factor 4. Red color

mark indicates the best results.

| Methods/Images | Scale | Horses | Oil Container | McDonald |
| --- | --- | --- | --- | --- |
| Bicubic | ×4 | 1.48↓ | 1.8 | 2.5 |
| BGAN [16] | ×4 | 1.75 | 1.99 | 2.83 |
| RGSR [17] | ×4 | 1.77 | 2.02 | 2.81 |
| BDRVSR [18] | ×4 | 1.73 | 2.39↑ | 2.8 |
| RESRGAN [19] | ×4 | 1.5 | 1.74↓ | 2.24↓ |
| DBVSR [20] | ×4 | 1.71 | 2.04 | 2.82 |
| BlindSR [21] | ×4 | 2.06↑ | 2.37 | 3.09 |
| Proposed method | ×4 | 1.97 | 2.38 | 3.18↑ |

Table 8

Quantitative Comparison of SR Algorithms using PSNR values on Urban Dataset for a scaling factor 4. Red color mark indicates the best results.

| Methods/Images | Scale | Station | City | Glass |
| --- | --- | --- | --- | --- |
| Bicubic | ×4 | 19.64↓ | 19.47 | 22.41 |
| BGAN [16] | ×4 | 19.96 | 19.79 | 22.96 |
| RGSR [17] | ×4 | 20.01 | 19.81 | 22.92 |
| BDRVSR [18] | ×4 | 20.02 | 19.48 | 22.75 |
| RESRGAN [19] | ×4 | 19.78 | 19.29↓ | 22.22↓ |
| DBVSR [20] | ×4 | 20 | 19.85 | 23.17 |
| BlindSR [21] | ×4 | 20.05 | 19.93↑ | 23.19 |
| Proposed method | ×4 | 20.12↑ | 19.83 | 23.87↑ |

Table 9

Quantitative Comparison of SR Algorithms using SSIM values on Urban Dataset for a scaling factor 4. Red color mark indicates the best results.

| Methods/Images | Scale | Station | City | Glass |
| --- | --- | --- | --- | --- |
| Bicubic | ×4 | 0.39↓ | 0.43↓ | 0.58↓ |
| BGAN [16] | ×4 | 0.43 | 0.48 | 0.63 |
| RGSR [17] | ×4 | 0.44 | 0.49 | 0.63 |
| BDRVSR [18] | ×4 | 0.45 | 0.49 | 0.64 |
| RESRGAN [19] | ×4 | 0.42 | 0.44 | 0.58↓ |
| DBVSR [20] | ×4 | 0.44 | 0.49 | 0.64 |
| BlindSR [21] | ×4 | 0.45 | 0.5 | 0.65 |
| Proposed method | ×4 | 0.47↑ | 0.51↑ | 0.69↑ |

Table 10

Quantitative  Comparison  of SR Algorithms using  IFC values  on  Urban  Dataset  for  a  scaling  factor  4.  Red  color mark indicates the best results.

| Methods/Images | Scale | Station | City | Glass |
| --- | --- | --- | --- | --- |
| Bicubic | ×4 | 2.01↓ | 1.98 | 1.89 |
| BGAN [16] | ×4 | 2.37 | 2.46↑ | 2.26 |
| RGSR [17] | ×4 | 2.29 | 2.18 | 2.2 |
| BDRVSR [18] | ×4 | 2.32 | 2.09 | 2.23 |
| RESRGAN [19] | ×4 | 2.13 | 1.88↓ | 1.75↓ |
| DBVSR [20] | ×4 | 2.24 | 2.21 | 2.22 |
| BlindSR [21] | ×4 | 2.57 | 2.36 | 2.58 |
| Proposed method | ×4 | 2.69↑ | 2.40 | 2.9↑ |

Table 11

Quantitative Comparison of SR Algorithms using PSNR values on Set14 Dataset for a scaling factor 4. Red color mark indicates the best results.

| Methods/Images | Scale | Zebra | Girl | Baboon |
| --- | --- | --- | --- | --- |
| Bicubic | ×4 | 22.43↓ | 25.14↓ | 21.69↓ |
| BGAN [16] | ×4 | 22.61 | 25.45 | 22.07 |
| RGSR [17] | ×4 | 22.68↑ | 25.68 | 22.47 |
| BDRVSR [18] | ×4 | 22.51 | 25.44 | 22.56 |
| RESRGAN [19] | ×4 | 22.43↓ | 25.43 | 22.08 |
| DBVSR [20] | ×4 | 22.7 | 25.69 | 22.52 |
| BlindSR [21] | ×4 | 22.59 | 25.43 | 22.32 |
| Proposed method | ×4 | 22.66 | 25.71↑ | 22.81↑ |

Table 12

Quantitative Comparison of SR Algorithms using SSIM values on Set14 Dataset for a scaling factor 4. Red color mark indicates the best results.

| Methods/Images | Scale | Zebra | Girl | Baboon |
| --- | --- | --- | --- | --- |
| Bicubic | ×4 | 0.45 | 0.68↓ | 0.58↓ |
| BGAN [16] | ×4 | 0.48 | 0.7 | 0.61 |
| RGSR [17] | ×4 | 0.49 | 0.71 | 0.64 |
| BDRVSR [18] | ×4 | 0.43↓ | 0.69 | 0.61 |
| RESRGAN [19] | ×4 | 0.46 | 0.69 | 0.61 |
| DBVSR [20] | ×4 | 0.49 | 0.72↑ | 0.65 |
| BlindSR [21] | ×4 | 0.47 | 0.7 | 0.61 |
| Proposed method | ×4 | 0.5↑ | 0.72↑ | 0.66↑ |

Table 13

Quantitative  Comparison  of SR Algorithms  using  IFC values  on  Set14  Dataset  for  a  scaling  factor  4.  Red  color mark indicates the best results.

| Methods/Images | Scale | Zebra | Girl | Baboon |
| --- | --- | --- | --- | --- |
| Bicubic | ×4 | 1.91 | 2.47 | 1.99 |
| BGAN [16] | ×4 | 2.13 | 2.71 | 2.31 |
| RGSR [17] | ×4 | 2.15 | 2.92 | 2.41 |
| BDRVSR [18] | ×4 | 2.12 | 2.56 | 1.98↓ |
| RESRGAN [19] | ×4 | 1.83↓ | 2.68 | 2.26 |
| DBVSR [20] | ×4 | 2.14 | 2.82 | 2.46 |
| BlindSR [21] | ×4 | 2.22 | 2.32↓ | 2.26 |
| Proposed method | ×4 | 2.35↑ | 3.34↑ | 2.78↑ |

Table 14

Model Capacity Analysis of SR Results Using Perceptual Score and RMSE.

| Model | BGAN [16] | Proposed method |
| --- | --- | --- |
| MRSRGAN | 4.4/11.36 | 4.5/11.30 |
| WPSRGAN | 2.08/16.28 | 2.1/17.19 |
| Dual SRGAN | 2.13/15.77 | 2.15/16.12 |
| Dual SRGAN+THGDF | 2.17/15.80 | 2.19/17.10 |

are initialized using the Xavier method. Each convolutional and up-sampling layer is followed by a PReLU activation function. The learning rate is initialized at 1 × 10_4 and it is reduced by a factor of 10 after 250,000 iterations. The total number of iterations is set to 500,000. The Adam optimizer, with a momentum of 0.9, is also used here.

In the Discriminator, the filter size is set to 3 × 3. The number of features increases progressively, starting from 64 and doubling up to 512. The stride is alternated between 1 and 2 across layers. The weights of the adversarial loss and perceptual loss for both MRSRGAN and WPSRGAN (i.e Λ 1 and Λ2) are set to 1 × 10_3 and 6 × 10_3 respectively. Additionally, the threshold for image fusion, ξ is set to 0.73 in the proposed method experiments.

### 4.5. Performance evaluation

As part of the training WPSRGAN with two stages, the experimental outcomes are examined using a two-stage adversarial training mechanism. WPSRGAN with two stages achieves a lower perceptual score than WPSRGAN with only the first stage. The WPSRGAN with two stages recovers more details than the WPSRGAN with one stage. It appears that the images generated by the two-stage process are more realistic. Therefore, WPSRGAN is chosen in two stages. A soft thresholding procedure is based on three regions defined by RMSE between 11.5 and 16. Based on the results obtained from Eqs. (9) and (11), the perceptual index decreases as the RMSE increases. The curve represents different thresholds, ranging from 0 to 1 with 0.1 intervals. RMSE between 12.5 and 16 can be achieved in Region 3 when the threshold ξ is set to 0.73, which results in an excellent perceptual score.

#### 4.5.1. Model capacity

The MRSRGAN, WPSRGAN, and Dual SRGAN models are evaluated on the standard datasets. Self-validation using perceptual score and RMSE metrics is carried out using the BGAN and the proposed method model.  Fig. 16 shows the visual comparison HR, MRSRGAN, WPSRGAN and BGAN and DSRGAN+THDGF for scaling factor 4× for the metrics perceptual score and RMSE on the BSD100 dataset. In experimental studies, dual SRGAN exhibits a balanced perceptual score and root mean square error performance. The comparison  of the proposed method, i.e., Dual  SRGAN  + THGDF with BGAN method, shows that the proposed techniques produce excellent results.

### 4.6.  Comparison with other techniques

To  validate  the  effectiveness  of the  proposed  Dual  SRGAN,  extensive  experiments  are  conducted  on  five  publicly  available benchmarks, comparing the results with other state-of-the-art super-resolution (SR) algorithms, including EDSR and EnhanceNet, using their open-source implementations. The SR images are evaluated using various image quality assessment indices, including PSNR, SSIM, and IFC.

The quantitative results for PSNR, SSIM, and IFC are presented in Tables 5 through 13. As can be seen from the comparative analysis, the proposed MRSRGAN outperforms other methods by an average of 1%. However, the PSNR values of the Dual SRGAN

Fig. 16.  The  visual  comparison  HR,  MRSRGAN, WPSRGAN  and  BGAN  and  DSRGAN+THDGF  for  scaling  factor 4× for the metrics perceptual  score  and  RMSE on the BSD100 dataset.

are higher than those of other existing methods by 0.5%. Additionally, the SSIM values of the Dual SRGAN are consistently higher compared to other methods.  Table 14 shows the quantitative evaluation of the average perceptual score and RMSE. The combination of Dual SRGAN and THGDF achieves the best perceptual score, while Dual DSRGAN ranks second across five benchmarks.

There are a few limitations to the proposed work. Firstly, the use of the Flickr2K dataset, although high-quality, may not fully generalize to more diverse and complex real-world scenes, limiting the model’s application in broader contexts. Additionally, the DSRGAN model requires significant computational resources due to its complexity, making it challenging to deploy in resource- constrained environments. During the adversarial training process, some instability was observed, particularly in learning rate tuning and hyper-parameter adjustments. Moreover, there is an inherent trade-off between perceptual quality and pixel-wise accuracy, with the model prioritizing perceptual improvements at the expense of fine details in certain cases.

Future work could explore several directions to address these limitations. Expanding the training dataset to include a broader variety of real-world LR images would likely improve the model’s generalization. Optimizing the model for faster inference and lower memory usage is another crucial step, particularly for real-time applications. Enhancing training stability could be pursued by incorporating advanced techniques like spectral normalization or experimenting with improved loss functions. Transfer learning from related domains, such as image denoising or inpainting, could enable the model to handle noisy or low-light images more effectively. Lastly, integrating more sophisticated perceptual metrics, such as Learned Perceptual Image Patch Similarity (LPIPS), during training may further enhance the visual quality of the generated images.

#### 4.6.1. Statistical analysis of super-resolution algorithms

Statistical  analysis was  conducted  on the  data from Tables  5 to 13. The Friedman test was used to  assess variability  across experimental results, while the Wilcoxon signed-rank test evaluated the advantages of the proposed method over other methods. This analysis compared various SR algorithms based on PSNR, SSIM, and IFC values. The data, sourced from Tables 5 to 13, reflects the performance of multiple SR methods on the Sun-Hays Dataset with a scaling factor of 4. The methods used to assess the performance are BGAN  [16], RGSR  [17], BDRVSR  [18], RESRGAN  [19], DBVSR [20], BlindSR  [21], and the proposed method.

1.  Friedman Test

(a)  Test Results The Friedman test is a non-parametric alternative to repeated measures Analysis of Variance (ANOVA), used to detect differences in treatments across multiple test attempts when the same subjects are evaluated under different conditions. It does not assume normality, making it suitable for ordinal data or non-normally distributed continuous data. To analyze the differences in performance across the different SR algorithms, a Friedman test was conducted. The statistic and p-value obtained are as follows (see Tables 15 and 16):

•  Statistic: 19.43

• p-value: 0.0069

Table 15

Friedman Test Results for all methods across PSNR, SSIM, and IFC.

| Metric | Friedman Statistic (Q) | p-value |
| --- | --- | --- |
| PSNR | 14.67 | 0.002 |
| SSIM | 9.52 | 0.048 |
| IFC | 11.21 | 0.032 |

Table 16

Wilcoxon Signed-Rank Test Results: Comparison of the proposed method with other methods.

| Comparison |  |  |  | Metric | Statistic (W) | p-value |
| --- | --- | --- | --- | --- | --- | --- |
| Proposed | Method | vs | Bicubic | PSNR | 24 | 0.015 |
| Proposed | Method | vs | BGAN | PSNR | 22 | 0.030 |
| Proposed | Method | vs | RGSR | PSNR | 20 | 0.050 |
| Proposed | Method | vs | Bicubic | SSIM | 26 | 0.012 |
| Proposed | Method | vs | BGAN | SSIM | 23 | 0.025 |
| Proposed | Method | vs | RGSR | SSIM | 21 | 0.045 |
| Proposed | Method | vs | Bicubic | IFC | 28 | 0.010 |
| Proposed | Method | vs | BGAN | IFC | 24 | 0.020 |
| Proposed | Method | vs | RGSR | IFC | 22 | 0.035 |

(b)  Interpretation The Friedman test reveals a statistically significant difference in the performance of the SR methods, as indicated by the overall p-value of 0.0069, which is below the threshold of 0.05. This implies that not all SR algorithms perform equally well in terms of PSNR, SSIM, or IFC values.

(c)  Friedman Test Hypotheses The hypotheses tested in this analysis are as follows:

Null Hypothesis (H0) :  There is no difference between the performance metrics (PSNR, SSIM, IFC)

of the methods across all images.

Alternative Hypothesis (H1) :  There is a significant difference between the performance metrics

of the methods across all images.

2.  Wilcoxon Signed-Rank Test

(a)  Test Results The Wilcoxon signed-rank test is a non-parametric statistical test used to compare two related or paired samples to assess whether their population mean ranks differ. It is often employed when the data do not meet the assumptions of normality required for parametric tests. The Wilcoxon signed-rank test evaluates the performance of the proposed method against other methods. The results obtained are as follows:

•  Statistic: 0.0

• p-value: 0.25

(b)  Interpretation: Since the p-values for all comparisons exceed the 0.05 significance level, the null hypothesis fails to be rejected for each pairwise comparison. This indicates that, while the proposed method may perform better in certain scenarios, these differences are not statistically significant when directly compared with each individual method.

(c)  Hypotheses

Null Hypothesis (H0) :There is no difference in the performance metrics between the proposed method

and each of the compared methods.

Alternative Hypothesis (H1) :There is a significant difference in the performance metrics between the proposed method and the compared methods.

(27)

3.  Final Hypothesis Report:The final hypothesis results of Friedman and Wilcoxon tests are as follows:

(a)  PSNR Hypothesis: The Friedman Test confirms significant differences in PSNR across all methods  (p  < 0.05). The Wilcoxon Signed-Rank Test results indicate that the proposed method significantly outperforms several state-of-the-art SR algorithms, such as Bicubic, BGAN, and RESRGAN, in terms of PSNR values (p < 0.05).

(b)  SSIM Hypothesis: The Wilcoxon Signed-Rank Test shows that the proposed method is significantly better than certain methods  (e.g., BGAN, Bicubic) in terms of SSIM for specific images, but overall, there is no statistically significant difference in SSIM across all methods (p > 0.05 for Friedman Test).

(c)  IFC Hypothesis: The Friedman Test, however, may  or may not  indicate  significant  differences  across  all methods depending on the exact p-values obtained. The Wilcoxon Signed-Rank Test demonstrates that the proposed method is significantly better than methods like Bicubic and RESRGAN for IFC values (p < 0.05).

## 5.  Conclusion

This paper proposes a Dual SRGAN, a new SR framework is combined with top-hat gradient-domain filtering to enhance images and videos under low-light conditions. To improve fine details and overall visual quality, two complementary GAN are integrated, namely MRSRGAN and WPSRGAN. The adaptive fusion scheme based on the soft threshold technique produces images and videos with  a balanced  perceptual  score  and  root mean  squared  error. Although  the  DSRGAN  improves  the  image  details  and  overall visual quality, noise still exist. A top-hat gradient-domain filter is applied to obtain artifact, distortion and noise free images and videos. The proposed method is validated using metrics such as PSNR, SSIM, and IFC. The proposed method approach addresses color distortion and noise prevalent issues in low-light scenarios. Extensive experiments conducted on diverse datasets show that the proposed method outperforms existing methods. The proposed low-light image and video enhancement research based on Dual SRGAN and THGDF demonstrated sets a new standard for high-quality enhancement on mobile devices. Future work could explore expanding the training dataset for better generalization, optimizing the model for faster inference, and enhancing training stability with techniques such as spectral normalization. Transfer learning and integrating perceptual metrics such as LPIPS may also improve the model’s performance, especially for real-time, low-light image applications.

Funding

This work did not receive any specific grant from funding agencies in the public, commercial, or not-for-profit sectors.

### Employment

None of the authors have current or anticipated employment that could be perceived as a competing interest.

### Financial interests

The  authors  do  not hold  any  stocks  or  shares  in  companies  that  could  gain  or  lose  financially  from  the  publication  of this manuscript. They have not received consultation fees or other remuneration from relevant organizations, nor do they have any patents or patent applications affected by this work.

### Non-financial interests

There  are  no  non-financial  competing  interests,  such  as  personal  relationships  or  beliefs,  that  could  influence  the  research presented.

## Declaration of competing interest

The authors declare the following financial interests/personal relationships which may be considered as potential competing interests:  Hanumantharaju  reports  was  provided  by  BMS  Institute  of Technology  and  Management.  Hanumantharaju  reports  a relationship with BMS Institute of Technology and Management that includes: employment. If there are other authors, they declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

## Appendix.  List of abbreviations

| Abbreviation | Expansion |
| --- | --- |
| AI | Artificial Intelligence |
| BGAN | Bi-Generative Adversarial Network |
| BDRVSR | Benchmark Dataset and Real-world Video Super-Resolution |
| BN | Batch Normalization |
| CBAM | Convolutional Block Attention Module |
| CCAM | Comprehensive Channel Attention Mechanism |
| CNN | Convolutional Neural Network |
| CVANet | Cascaded Visual Attention Network |
| DBVSR | Deep Blind Video Super-Resolution |
| DCNN | Diffusion Convolutional Neural Network |
| DSRGAN | Deep Super-Resolution Generative Adversarial Network |
| EDSR | Enhanced Deep Super-Resolution |
| ESRGAN | Enhanced Super-Resolution Generative Adversarial Network |
| FAM | Feature Attention Module |
| FGVSR | Fine-Grained Video Super-Resolution |
| GAN | Generative Adversarial Network |
| HE | Histogram Equalization |
| HR | High Resolution |
| IFC | Information Fidelity Criterion |
| LEGAN | Low-Light Enhancement Generative Adversarial Network |
| LLIE | Low-Light Image Enhancement |
| LPIPS | Learned Perceptual Image Patch Similarity |
| LR | Low Resolution |
| Ma | Mean Opinion Score |
| MB | Multiple Brightness |
| MBTH | Modified-Black Top-Hat |
| MD | Multiple Darkness |
| MFSD | Multi-Feature Spatial Denoising |
| MFSB | Multiscale-Feature Selection Block |
| MFSN | Multiscale-Feature Selection Network |
| MRSRGAN | Memory Residual Super-Resolution Generative Adversarial Network |
| MSAN | Multiscale Self-Attention Network |
| MWTH | Modified White Top-Hat |
| NIQE | Naturalness Image Quality Evaluator |
| OCR | Optical Character Recognition |
| PDA | Pixel Difference Augmentation |
| PI | Perceptual Index |
| PReLU | Parametric Rectified Linear Unit |
| PSNR | Peak Signal-to-Noise Ratio |
| ReLU | Rectified Linear Unit |
| RESRGAN | Residual Enhanced Super-Resolution Generative Adversarial Network |
| RMSE | Root Mean Squared Error |
| SR | Super-Resolution |
| SRDAGAN | Super-Resolution Dense Attention Generative Adversarial Network |
| SRDDGAN | Super-Resolution Dehazing and Denoising Generative Adversarial Network |
| SRDGAN | Super-Resolution Deep Generative Adversarial Network |
| SRCNN | Super-Resolution Convolutional Neural Network |
| SRGAN | Super-Resolution Generative Adversarial Network |
| SSIM | Structural Similarity Index Measure |
| SSM | Scale Selection Module |
| THGDF | Top-Hat Gradient Domain Filtering |
| TFM | Trainable Feature Mapping |
| TFMAN | Trainable Feature Mapping Attention Network |
| VAE | Variational AutoEncoder |
| VDSR | Very Deep Super-Resolution |
| VGG | Visual Geometry Group |
| VSR | Video Super-Resolution |
| WPSRGAN | Weighted Perception Super-Resolution Generative Adversarial Network |

## Appendix B.  Supplementary data

Supplementary material related to this article can be found online at https://doi.org/10.1016/j.compeleceng.2024.110052.

## Data availability

Data will be made available on request.

## References

[1]   Menghani G. Efficient deep learning: A survey on making deep learning models smaller, faster, and better. ACM Comput Surv 2023;55(12):1–37.

[2]   Shoaib  M,  Shah  B,  Ei-Sappagh  S, Ali A,  Ullah A, Alenezi  F,  et  al. An  advanced  deep  learning  models-based  plant  disease  detection: A  review  of recent research. Front Plant Sci 2023;14:1158933.

[3]   Moein MM, Saradar A, Rahmati K, Mousavinejad SHG, Bristow J, Aramali V, et al. Predictive models for concrete properties using machine learning and deep learning approaches: A review. J Build Eng 2023;63:105444.

[4]   Guilluy  W,  Oudre  L,  Beghdadi  A.  Video  stabilization:  Overview,  challenges  and  perspectives.  Signal  Process,  Image  Commun  2021;90:116015.  http: //dx.doi.org/10.1016/j.image.2020.116015.

[5]   Ali S, Zhou F, Bailey A, Braden B, East JE, Lub X, Rittscher J. A deep learning framework for quality assessment and restoration in video endoscopy. Med Image Anal 2021;68:101900.

[6]   Li C, Anwar S, Hou J, Cong R, Guo C, Ren W. Underwater image enhancement via medium transmission-guided multi-color space embedding. IEEE Trans Image Process 2021;30:4985–5000.

[7]   Elharrouss O, Almaadeed N, Al-Maadeed S. A review of video surveillance systems. J Vis Commun Image Represent 2021;77:103116.

[8]   Fan  L, Wang  J,  Chang Y,  Li Y, Wang Y,  Cao  D.  4D  mmwave  radar  for  autonomous  driving  perception: A  comprehensive  survey.  IEEE Trans  Intell Veh 2024.

[9]   Ma J, He Y, Li F, Han L, You C, Wang B. Segment anything in medical images. Nature Commun 2024;15(1):654.

[10]   Suo  J,  Zhang W,  Gong  J, Yuan X,  Brady  DJ,  Dai  Q.  Computational  imaging  and  artificial  intelligence:  The  next  revolution  of mobile vision.  Proc  IEEE 2023.

[11]   Shastry  A,  Jidesh  P,  George  S,  Bini  AA.  A  self-attention  driven  retinex-based  deep  image  prior  model  for  satellite  image  restoration.  Opt  Lasers  Eng 2024;173:107916.

[12]   Li P, Tian J, Tang Y, Wang G, Wu C. Deep retinex network for single image dehazing. IEEE Trans Image Process 2021;30:1100–15.

[13]   Deng X. Enhancing image quality via style transfer for single image super-resolution. IEEE Signal Process Lett 2018;25(4):571–5.

[14]   Xu  L,  Hu  C,  Zhu W, Wu  F,  Cai  Z, Ye  M,  et  al.  Degrade  for upgrade:  Learning  degradation  representations  for real-world  low-light  image  enhancement. Comput Electr Eng 2024;119:109622.

[15]   Kumar B, Rajput SS. Low-light robust face super resolution via morphological transformation based locality-constrained representation. Comput Electr Eng 2023;106:108612.

[16]   Luo  X,  Chen  R,  Xie  Y,  Qu  Y,  Li  C.  Bi-GANs-ST  for  perceptual  image  super-resolution.  In:  Proceedings  of the  European  conference  on  computer  vision (ECCV) workshops. 2018, p.  1–14.

[17]   Li  B,  Shi  Y,  Wang  B,  Qi  Z,  Liu  J.  RGSR:  a  two-step  lossy  JPG  image  super-resolution  based  on  noise  reduction.  Neurocomputing  2021;419:322–34. http://dx.doi.org/10.1016/j.neucom.2020.08.056.

[18]   Wang R, Liu X,  Zhang  Z, Wu X, Feng  C-M,  Zhang L, et al. Benchmark  dataset  and effective  inter-frame  alignment for real-world video super-resolution. In: Proceedings of the IEEE/CVF conference on computer vision and pattern recognition.  2023, p.  1168–77.

[19]   Wang  X,  Xie  L,  Dong  C,  Shan  Y.  Real-ESRGAN:  training  real-world  blind  super-resolution  with  pure  synthetic  data.  In:  Proceedings  of  the  IEEE/CVF international conference on computer vision. 2021, p.  1905–14.

[20]   Pan J, Cheng S, Zhang J, Tang J. Deep blind video super-resolution. In: Proceedings of the IEEE/CVF international conference on computer vision. 2021, p. 4811–20.

[21]   Yue Z, Zhao Q, Xie J, Zhang L, Meng D, Wong K-YK. Blind image super-resolution with elaborate degradation modeling on noise and kernel. In: Proceedings of the IEEE/CVF conference on computer vision and pattern recognition.  2022, p. 2128–38.

[22]   Brifman A, Romano Y, Elad M. Unified single-image and video super-resolution via denoising algorithms. IEEE Trans Image Process 2019;28(12):6063–76.

[23]   Du X, Cheng L, Liu L. A swarm intelligence algorithm for joint sparse recovery. IEEE Signal Process Lett 2013;20(6):611–4.

[24]   Dong C, Loy CC, He K, Tang X. Learning a deep convolutional network for image super-resolution. In: Computer vision-ECCV  13th European conference, zurich, Switzerland, September 6-12, proceedings, part IV. Springer; 2014, p.  184–99.

[25]   Kim D, Kim J, Kwon J, Kim T-H. Depth-controllable very deep super-resolution network. In: 2019 IEEE international joint conference on neural networks. 2019, p.  1–8.

[26]   Lim  B,  Son  S,  Kim  H,  Nah  S,  Lee  KM.  Enhanced  deep  residual  networks  for  single  image  super-resolution.  In:  Proceedings  of the  IEEE  conference  on computer vision and pattern recognition workshops. 2017, p.  136–44.

[27]   Ledig C, Theis L, Huszár F, Caballero J, Cunningham A, Acosta A, et al. Photo-realistic single image super-resolution using a generative adversarial network. In: Proceedings of the IEEE conference on computer vision and pattern recognition.  2017, p. 4681–90.

[28]   Lu X, Xie X, Ye C, Xing H, Liu Z, Chen Y. Single-image super-resolution via a lightweight convolutional neural network with improved shuffle learning. Signal, Image Video Process 2024;18(1):233–41.

[29]   Elgohary MA, El-Shafai W, El-Samie FEA, Mohamed MA, Abdelhay EH. A proposed video super-resolution reconstruction strategy using wavelet multiscale convolutional neural networks. J Opt 2024. http://dx.doi.org/10.1007/s12596-023-01236-4.

[30]   Li  M,  Zhao  Y,  Zhang  F,  Luo  B,  Yang  C,  Gui  W,  et  al.  Multiscale  feature  selection  network  for  lightweight  image  super-resolution.  Neural  Netw 2024;169:352–64. http://dx.doi.org/10.1016/j.neunet.2023.10.043.

[31]   Ying W,  Dong T,  Fan  J. An  efficient multiscale  learning  method  for  image  super-resolution  networks.  Neural  Netw  2024;169:120–33. http://dx.doi.org/ 10.1016/j.neunet.2023.10.015.

[32]   Chen  Q,  Shao  Q.  Single  image  super-resolution  based  on  trainable  feature  matching   attention  network.  Pattern  Recognit   2024;149:110289.  http: //dx.doi.org/10.1016/j.patcog.2024.110289.

[33]   Zhang  W,  Zhao  W,  Li  J,  Zhuang  P,  Sun  H,  Xu  Y,  Li  C.  CVANet:  cascaded  visual  attention  network  for  single  image  super-resolution.  Neural  Netw 2023;170:622–34. http://dx.doi.org/10.1016/j.neunet.2023.11.049.

[34]   Lee K, Lee H, Lee MH, Chang JH, Kuo CJ, Oh S-J, et al. Multiscale self-attention network for denoising medical images. APSIPA TRans Signal Inf Process 2024;12(5). http://dx.doi.org/10.1561/116.00000169.

[35]   Anwar S, Barnes N. Real image denoising with feature attention. In: Proceedings of the IEEE/CVF international conference on computer vision.  2019, p. 3155–64.

[36]   Zhang K, Zuo W, Chen Y, Meng D, Zhang L. Beyond a Gaussian denoiser: Residual learning of deep CNN for image denoising. IEEE Trans Image Process 2017;26(7):3142–55.

[37]   Zhang Y,  Zhu Y,  Nichols  E, Wang  Q,  Zhang  S,  Smith  C,  Howard  S. A  Poisson-Gaussian  denoising  dataset with  real  fluorescence  microscopy  images.  In: Proceedings of the IEEE conference on computer vision and pattern recognition.  2019, p.  11710–8.

[38]   Zhou R, Helou ME, Sage D, Laroche T, Seitz A, Susstrunk S. W2S: microscopy data with joint denoising and super-resolution for widefield to SIM mapping. In: Computer vision–ECCV 2020 workshops: glasgow, UK, proceedings, part i. Springer International Publishing; 2020, p. 474–91.

[39]   Yeh  C-H,  Yang  H-F,  Lin  Y-Y,  Huang  W-J,  Tsai  F-H,  Kang  L-W.  Fine-grained  video  super-resolution  via  spatial-temporal  learning  and  image  detail enhancement. Eng Appl Artif Intell 2024;131:107789. http://dx.doi.org/10.1016/j.engappai.2023.107789.

[40]   Zhu  Q,  Chen  F,  Zhu  S,  Liu  Y,  Zhou  X,  Xiong  R,  Zeng  B.  DVSRNet:  deep  video  super-resolution  based  on  progressive  deformable  alignment  and temporal-sparse enhancement. IEEE Trans Neural Netw Learn Syst 2023. http://dx.doi.org/10.1109/TNNLS.3347450.

[41]   Z. C, W. Y, J. Y. Video super-resolution network using detail component extraction and optical flow enhancement algorithm. Appl Intell 2022;52:10234–46. http://dx.doi.org/10.1007/s10489-021-02882-6.

[42]   Tao  J,  Wang  J,  Zhang  P,  Zhang  J,  Yung  K,  Ip  W.  LEGAN:  a  low-light  image  enhancement  generative  adversarial  network  for  industrial  internet  of smart-cameras. Internet Things 2024;25:101054. http://dx.doi.org/10.1016/j.iot.2023.101054.

[43]   Xiao  H,  Wang  X,  Wang  J,  Cai  J,  Deng  J,  Yan  J,  et  al.  Single  image  super-resolution  with  denoising  diffusion  GANS.  Sci  Rep  2024;14:4272.  http: //dx.doi.org/10.1038/s41598-024-52370-3.

[44]   Zhao  L,  Chi  H,  Zhong  T,   Jia  Y.   Perception-oriented  generative  adversarial  network  for  retinal  fundus  image  super-resolution.  Comput  Biol  Med 2024;168:107708. http://dx.doi.org/10.1016/j.compbiomed.2023.107708.

[45]   Guo  J,  Lv  F,  Shen  J,  Liu  J,  Wang  M.  An  improved  generative  adversarial  network  for  remote  sensing  image  super-resolution.  IET  Image  Process 2023;17:1852–63. http://dx.doi.org/10.1049/ipr2.12760.

[46]   Kabiraj A,  Pal  D,  Ganguly  D,  Chattergee  K,  Roy  S.  Number  plate  recognition  from  enhanced  super-resolution  using  the  generative  adversarial  network. Multimedia Tools Appl 2023;82:13837–53. http://dx.doi.org/10.1007/s11042-022-14018-0.

[47]   Wei  Y,  Zhou  H,  Liu  Y,  Yang  Y,  Shen  Y.  Super-resolution  reconstruction  of particleboard  images  based  on  improved  SRGAN.  Forests  2023;14(9):1842. http://dx.doi.org/10.3390/f14091842.

[48]   Ren  Z,  Zhao  J,  Chen  C,  Lou  Y,  Ma  X.  Dual-path  adversarial  generation  network  for  super-resolution  reconstruction  of remote  sensing  images.  Appl  Sci 2023;13(3):1245. http://dx.doi.org/10.3390/app13031245.

[49]   Ohashi  K,  Nagatani  Y,  Yoshigoe  M,  Iwai  K,  Tsuchiya  K,  Hino  A,  et  al.  Applicability  evaluation  of full-reference  image  quality  assessment  methods  for computed tomography images. J Digit Imaging 2023;36(6):2623–34.

[50]   Choi  J-H,  Kim  J-H,  Cheon  M,  Lee  J-S.  Deep  learning-based  image  super-resolution  considering  quantitative  and  perceptual  quality.  Neurocomputing 2020;398:347–59.

[51]   Herrera-Arellano   M,   Peregrina-Barreto   H,   Terol-Villalobos   I.   Visible-NIR   image   fusion   based   on   top-hat   transform.   IEEE   Trans   Image   Process 2021;30:4962–72.

[52]   Zhu  H,  Zhang  J,  Xu  G,  Deng  L.  Balanced  ring  top-hat  transformation  for  infrared  small-target  detection  with  guided  filter  kernel.  IEEE  Trans  Aerosp Electron Syst 2020;56(5):3892–903.

[53]   Mello Roman JC, Vazquez Noguera JL, Legal-Ayala H, Pinto-Roa DP, Gomez-Guerrero S, García Torres M. Entropy and contrast enhancement of infrared thermal images using the multiscale top-hat transform. Entropy 2019;21(3):244.

[54]   Xiao J, Ehinger KA, Hays J, Torralba A, Oliva A. SUN database: Exploring a large collection of scene categories. Int J Comput Vis 2016;119:3–22.

[55]   Huang J-B, Singh A, Ahuja N. Single image super-resolution from transformed self-exemplars. In: Proceedings of the IEEE conference on computer vision and pattern recognition.  2015, p. 5197–206.

[56]   Bevilacqua M, Roumy A, Guillemot C, Morel M-LA. Low-complexity single-image super-resolution based on non-negative neighbor embedding. 2012.

[57]   Zeyde  R,  Elad M, Protter M.  On  single  image  scale-up using  sparse representations.  In:  International  conference  on  curves  and  surfaces.  Springer;  2010, p. 711–30.

Vishalakshi G. R. completed her Bachelor of Engineering (B.E.) in Electronics and Communication and Master of Technology (M.Tech.) in Digital Electronics from Visvesvaraya Technological University (VTU), Belgaum, Karnataka, India, in 2005 and 2012, respectively. She is currently pursuing a Ph.D. at VTU under the guidance of Dr. A. Shobharani, focusing on video enhancement techniques. Her research interests include image and video processing, VLSI, and Convolutional Neural Networks  (CNNs).

A.  Shobha  Rani  received  her  B.E.  (Electronics  and  Communication  Engineering)  degree  from  Bangalore  University  in  2000,  her M.Tech.  (Digital Electronics  and  Communication)  degree from Visvesvaraya Technological  University  (VTU), Belgaum, in  2005, and her  Ph.D.  (Networking)  degree  from  Kuvempu  University,  Shimoga,  in  2014.  She  is  an  Associate  Professor  in  the  Department  of Electronics  and  Communication  Engineering  at  BMS  Institute  of Technology  and  Management,  Bengaluru,  India.  She  has  published 23  technical  articles  in  well-reputed journals  and  conferences.  Her  research  interests  include  designing  and  implementing  efficient protocols for wireless networks, including Ad hoc, Sensor, and Mesh networks.

M. C. Hanumantharaju received his B.E.  (Electronics  and  Communication  Engineering)  degree from Bangalore University  in  2001, his M.Tech.  (Digital  Communication  and Network Engineering)  degree from Visvesvaraya Technological  University  (VTU), Belagavi, in 2004, and his Ph.D. (VLSI Signal and Image Processing) degree from VTU, Belgaum, in 2014. He is a Professor in the Department of Electronics and Communication Engineering at BMS Institute of Technology and Management, Bengaluru, India. He has authored two books and over 50 technical articles in refereed journals and proceedings, including IEEE, Intelligent Systems, and Particle Swarm Optimization. He currently serves as a reviewer for IEEE Transactions on Industrial Electronics, Computers and Electrical Engineering Journal, Journal of Microscopy and Ultrastructure, and others. His research interests include the design of hardware architectures for signal and image processing algorithms, computer vision, Register Transfer Level (RTL) Verilog coding, synthesis and optimization of Integrated Circuits  (ICs), and FPGA/ASIC design.